# CPCSHP3009 Demolish shopfronts and bulkheads

Release: 3

CPCSHP3009 Demolish shopfronts and bulkheads

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Performance Criteria numbering rectified and Modification History rectified. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0 Supersedes and is equivalent to CPCCSH3009 Demolish shopfronts and bulkheads. |

# Application

This unit of competency specifies the skills and knowledge required to demolish sections of existing shop buildings and to dispose of demolished materials and waste. It includes completing the required demolition work to schedule and following specific plans and specifications.

It applies to shopfitters who demolish old shop building components to make alterations, extensions and additions to the shop. Work may be completed onsite concurrently with other shopfitting work that must be scheduled to accommodate client requirements, the shop when it is operational, and the requirements of other shopfitters and associated contractors.

This demolition work is limited to non-load-bearing shop building structures, including:

external walls and cladding

suspended ceiling structures

shopfronts and glazed sections

joinery and fixtures

high level bulkheads.

Individuals at this level take responsibility for organising and completing these tasks with a high degree of self-direction.

Licensing, legislative or certification requirements may apply to this unit of competency in some states and territories. Relevant state and territory regulatory authorities should be consulted to confirm those requirements.

Completion of the general construction induction training program, specified in the Safe Work Australia model Code of Practice: Construction Work, is required by anyone carrying out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

# Pre-requisite Unit

Nil.

# Unit Sector

Shopfitting

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan shop demolition work. | 1.1 | Identify, read and interpret shopfront demolition details from plans and specifications, and confirm against job requirements. |
|  |  | 1.2 | Interpret and apply organisational quality assurance requirements appropriate for scope of work. |
|  |  | 1.3 | Determine designated demolition area of shopfront and bulkhead, identify hazards and assess risks. |
|  |  | 1.4 | Establish, and confirm with team members, risk management and hazard control measures and manage the status of existing services. |
|  |  | 1.5 | Apply and document health, safety and environmental requirements for the worksite and for individual demolition tasks. |
|  |  | 1.6 | Sequence and schedule demolition tasks in consultation with team members, others onsite and client, to ensure work is completed within given timeframe. |
|  |  | 1.7 | Select, fit and use personal protective equipment (PPE) required for the task. |
|  |  | 1.8 | Select plant, tools and equipment suited to the demolition work, check for serviceability, and rectify and report faults. |
|  |  | 1.9 | Arrange and organise scaffolding and lifting equipment to meet job requirements. |
|  |  | 1.10 | Plan the handling, removal and disposal of identified demolition materials to meet work health and safety (WHS) and environmental requirements. |
| 2 | Prepare demolition worksite. | 2.1 | Erect hoardings, barricades and signage as specified in work instructions. |
|  |  | 2.2 | Erect and check scaffolding, fall protection and props to ensure they are installed against demolition plan. |
|  |  | 2.3 | Set out and mark building components to be demolished and removed. |
|  |  | 2.4 | Check site preparation and safety meet workplace safety and environmental requirements before starting work. |
|  |  | 2.5 | Cover and protect existing structure, furniture and equipment from dust and damage. |
|  |  | 2.6 | Identify, isolate or disconnect services for safety and to avoid damage. |
| 3 | Apply demolition procedures. | 3.1 | Assess individual tasks immediately before demolition work to identify potential hazards, and review and amend safety procedures in consultation with relevant personnel. |
|  |  | 3.2 | Install propping required to prevent collapse of overhead non-load-bearing elements. |
|  |  | 3.3 | Follow demolition procedures for safely dismantling and demolishing building structure. |
|  |  | 3.4 | Handle and remove demolished materials using appropriate handling techniques from location to designated storage area or skip bin. |
|  |  | 3.5 | Handle, store and stack demolished materials and components identified for salvage ready for reuse or transport. |
|  |  | 3.6 | Maintain effective verbal and non-verbal communication, including the use of mobile technology, with team members and others onsite to monitor and manage demolition procedures. |
| 4 | Clean up. | 4.1 | Clear work area and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 4.2 | Clean and maintain tools and equipment and rectify and report faults. |
|  |  | 4.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCSH3009 Demolish shopfronts and bulkheads.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCSHP3009 Demolish shopfronts and bulkheads

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Performance Criteria numbering rectified and Modification History rectified. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0 Supersedes and is equivalent to CPCCSH3009 Demolish shopfronts and bulkheads. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by dismantling and demolishing a shopfront and internal cabinet.

In doing this, the candidate must:

dismantle and demolish a shopfront of a minimum of 3.6 metres high and 3.6 metres wide, including a bulkhead, glazing, door closure and joinery

dismantle one internal cabinet or fixture of a minimum of 3.6 metres high and 2.4 metres wide, which is attached to the building structure

protect surrounding structure, floor covering, existing furniture and fixtures from damage

plan and schedule the demolition work to maintain continuity of business operations.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

shopfitting regulations, codes and standards relating to shopfront and bulkhead demolition

organisation’s quality assurance requirements relating to demolition of shopfronts and bulkheads

sustainable shopfitting practices

construction and demolition terminology used in shopfitting

content of and terms, symbols and conventions used in working drawings and specifications

characteristics, applications and limitations of materials used in shopfitting components:

acrylic

glass

laminates

manufactured board

solid timber

procedures and techniques relevant to demolishing shopfronts:

calculating volumes of material for removal

non-load-bearing external walls

suspended ceilings

shopfronts and glazed sections

joinery and fixtures

high-level bulkheads

maintaining operation of existing services

workplace safety:

job safety and environmental analysis (JSEA)

safe work method statements (SWMSs)

hierarchy of control

how to identify potential hazards

safety data sheets (SDSs)

hazardous manual tasks

exposure to excessive noise

removal of asbestos

exposure to dust

hazardous substances:

lead

fibreglass

working at heights

falling objects

vibration

electrical safety

emergency procedures (evacuation, location and use of firefighting equipment, first aid)

safety of public, visitors and others in the workplace

functional and operational features of tools and equipment:

air compressors and pneumatic tools

angle grinders

elevated work platforms (EWPs)

scaffolding

power saws

power drills.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to:

relevant shopfitting regulations, codes and Australian Standards

current workplace procedures, workplace safety and environmental documentation relating to demolition

shopfitting drawings and specifications, organisational policies, procedures and equipment to undertake the performance criteria and assessment requirements

digital technology, devices and software to communicate with others and to develop and save documents electronically.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad