# Assessment Requirements for CPCSIL3001 Work with products and materials containing crystalline silica

Release: 1

Assessment Requirements for CPCSIL3001 Work with products and materials containing crystalline silica

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 8.0. Newly created unit. |
| --- | --- |

# Performance Evidence

A person demonstrating competency in this unit must satisfy the requirements of the elements, performance criteria and foundation skills of this unit in addition to the specific performance and knowledge evidence described below.

Candidates must be able to:

demonstrate safe use of personal protective equipment (PPE) and respiratory protective equipment (RPE), including fit checking of disposable and re-usable devices for three different work activities in accordance with manufacturer instructions

demonstrate three different work activities, such as cutting, drilling or grinding products or materials containing crystalline silica (CS) applying all relevant work health and safety (WHS) considerations, including dust capture and suppression measures, PPE and/or RPE

demonstrate cleaning and disposal of contaminated products and work area in accordance with WHS procedures.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

hierarchy of risk control measures:

elimination

substitution

isolation

engineering controls

administrative controls

PPE/RPE

WHS risks working with products and materials containing CS, including features, commonalities and differences between CS and respirable crystalline silica (RCS)

products containing CS, such as:

engineered stone

concrete

mortar

bricks

pavers

tiles

natural stone

cement sheeting

aerated concrete

silica dust generated by workplace mechanical processes, such as:

cutting

crushing

drilling

grinding

cutting, grinding and/or abrasive polishing of engineered stone

additional work activities that can generate respirable silica dust particles, including:

excavation, earth moving and drilling plant operations

demolition

clay and stone processing machine operations

paving and surfacing

mining, quarrying and mineral ore treating processes

tunnelling

construction labouring activities

brick, concrete or stone cutting; especially using dry methods

abrasive blasting (blasting agent must not contain greater than 1 per cent of CS)

foundry casting

angle grinding, jack hammering and chiselling of concrete or masonry

hydraulic fracturing of gas and oil wells

pottery making

worksites where silica dust can be found:

manufacturing environment

residential construction

commercial construction

civil construction

AS/NZ 1715:2009 Selection, use and maintenance of respiratory protective equipment and AS/NZ 1716:2012 Respiratory protective devices, or their equivalent

Relevant Australian Standards, regulations and codes of practice

WHS procedures

relevant legislation governing handling and working with products containing CS, including monitoring requirements

employer responsibilities and duty of care requirements:

consultation and communication processes

health monitoring

personal exposure monitoring

use of PPE and compliant RPE, including disposable and re-usable devices in accordance with AS/NZ 1715:2009 Selection, use and maintenance of respiratory protective equipment and AS/NZ 1716:2012 Respiratory protective devices, or equivalent

RPE:

half-face disposable

half-face reusable

full-face reusable

powered air purifying respirators

factors affecting fit of PPE and RPE e.g., beards and facial hair

range of compliant plant, tools and equipment used with products containing CS

relevant documents with information on safe work practices and technical information on products containing CS, such as:

WHS policies and procedures

relevant jurisdictional legislation

relevant codes of practice

safe work method statements (SWMS)

job safety and environmental analysis (JSEA)

safety data sheets (SDS)

product manuals

technical specifications

product labels

maintenance requirements on respirators for RCS and for power tools, plant and equipment

safe cleaning and housekeeping methods with silica dust, including cleaning and managing wet or dry material on clothing

dust control measures.

# Assessment Conditions

Assessment of performance must be undertaken in the workplace or simulated workplace environment. A simulated workplace is one that fully replicates the resources, environment and any time and productivity pressures that exist in the actual workplace, and which meets industry standards for safety and environmental practices.

Candidates must be provided with access to:

real or simulated worksites

environmental requirements and sustainability principles

relevant tasks or work specifications

appropriate documents, materials, tools, equipment, PPE and P2 half-face disposable and re-usable RPE currently approved for use in industry and worksite

relevant codes of practice, Australian Standards, legislation, regulations and requirements of workplace policies and procedures as required by Commonwealth, state and territory regulators.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad