# Assessment Requirements for CPPCLO3015 Treat complex carpet stains

Release: 2

Assessment Requirements for CPPCLO3015 Treat complex carpet stains

# Modification History

| Release | Comment |
| --- | --- |
| Release 1 | Replaces superseded equivalent CPPCCL3015A Perform advanced stain removal. This version first released with CPP Property Services Training Package Version 4. |
| Release 2 | Minor update to typo in the Knowledge Evidence, replace ‘distilling’ with ‘diluting’. This version released with CPP Property Services Training Package Version 5. |

# Performance Evidence

A person demonstrating competency in this unit must satisfy the requirements of the elements, performance criteria, foundation skills and range of conditions of this unit.

The person must also remove three different complex stains from two different carpets as follows:

at least one complex stain from one of the following natural fibre carpets:

organic (plant) fibres, such as cotton and sisal

wool

at least one complex stain from one of the following synthetic fibre carpets:

acrylic

nylon

polyester

polypropylene.

The three different complex stains above must be selected from the following:

beverage

blood

candle wax

dye stuffs

food

human and animal waste

lipstick and makeup

medicine

nail polish

oil

plant stains

shoe polish

tar

timber stains.

In doing the above work, the person must also:

identify site hazards and control risks before commencing the task

correctly identify the type and characteristics of the carpet and carpet fibres

classify the type of stain or spot and identify cleaning techniques, cleaning equipment and chemicals, required for the task

use each of the following cleaning methods:

bonnet cleaning

dry absorbent compound

dry foam shampoo

hot water extraction

wet foam shampoo

correctly and safely handle waste.

# Knowledge Evidence

A person demonstrating competency in this unit must demonstrate knowledge of:

environmental requirements to be applied when treating complex carpet stains, including:

clean-up, containment and isolation procedures

environmental protection agency requirements relating to treating complex carpet stains

use of low-energy carpet-cleaning methods

use of low-moisture cleaning methods

use of low water-use equipment and other water-efficient cleaning methods

use of non-chemical carpet-cleaning methods

carpet characteristics, including:

carpet types

features of fibres

cleaning methods for carpets, including:

heat transfer process

pre-spotting and stain-removal methods

testing methods

water-efficient cleaning methods

correct techniques for performing:

extraction cleaning

heat transfer

treatments using:

oxidising agents, such as bleaches

reducing agents, such as dye strippers

key requirements of legislation, regulations, codes of practice and industry advisory standards relating to treating complex carpet stains, including AS/NZS 3733 Textile floor coverings – Cleaning maintenance of residential and commercial carpeting

processes for safely handling and disposing of waste, including:

chemicals past their expiry date

machine exhaust emissions

obsolete equipment

used containers

used or contaminated personal protective equipment (PPE)

used or unused chemicals

safe handling techniques for working with hazardous chemicals, including:

emergency chemical spill control measures

routes of entry and potential symptoms of exposure to chemicals

safe methods for diluting chemicals

working according to safety data sheets (SDS).

# Assessment Conditions

The following must be present and available to learners during assessment activities:

equipment:

as listed in the range of conditions

PPE as listed in the range of conditions

materials:

cleaning chemicals as listed in the range of conditions

specifications:

AS/NZS 3733 Textile floor coverings – Cleaning maintenance of residential and commercial carpeting

equipment operating manuals

product labels

SDS

physical conditions:

suitable work site or venue with complex carpet stains

relationships with team members and supervisor:

work may be conducted alone or as part of a team

relationship with client:

communication with the client is required to confirm and clarify requirements.

Timeframe:

as specified by the task.

### Assessor requirements

As a minimum, assessors must satisfy the assessor requirements in the Standards for Registered Training Organisations (RTOs) current at the time of assessment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b