# CPPCLO4105 Apply odour control to structures and contents

Release: 1

CPPCLO4105 Apply odour control to structures and contents

# Modification History

| Release 1 | This version first released with CPP Property Services Training Package Release 15.0. Supersedes and is not equivalent to CPPCLO3010 Apply odour control to carpets. |
| --- | --- |

# Application

This unit specifies the skills and knowledge required to remove odours from building structures such as timber and plaster, and contents including carpets, furnishings and personal items by identifying the type of odour and selecting and applying odour control methods according to the odour and material type. The source of odours can be due to factors such as animal or human body odour, animal waste, chemicals, decaying matter, faeces, fire and smoke, urine, vomit and water damage.

The unit is informed by aspects of standards set by the Institute of Inspection, Cleaning and Restoration Certification (IICRC) and relevant Australian Standards and applies to experienced technicians responding to jobs in commercial and domestic sites.

This unit is suitable for cleaning industry personnel who use well-developed cognitive, technical and communication skills and a broad knowledge base to select and apply specialist methods, tools, materials and information in disaster cleaning and restoration contexts. They complete routine and non-routine activities and provide and transmit solutions to a variety of predictable and sometimes unpredictable problems.

All work must be carried out in accordance with relevant state/territory legislation and regulations, including work health and safety (WHS), and codes of practice as well as organisational procedures.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil.

# Unit Sector

Cleaning

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Inspect and assess structures and contents affected by odour. | 1.1 | Clarify site access, areas to be restored and scope of work with building owner, client, insurer or supervisor. |
|  |  | 1.2 | Select, check for fit and serviceability, and use personal protective equipment (PPE) according to WHS and job requirements. |
|  |  | 1.3 | Identify site hazards, risks and contamination and determine control measures, including containment and engineering controls required, according to WHS requirements. |
|  |  | 1.4 | Determine the origin and type of odour according to job requirements. |
|  |  | 1.5 | Investigate materials and construction of affected structure and contents to determine if disposal is required or restoration is viable. |
|  |  | 1.6 | Seek specialist advice, where required, according to job, organisational and legislative requirements. |
|  |  | 1.7 | Document condition of area to be restored and advise client, building owner or insurer of the results of the assessment, proposed remediation methods, risks and likely outcomes and seek approval to proceed according to organisational requirements. |
| 2 | Prepare to apply odour control. | 2.1 | Determine odour control methods, equipment and sequence of tasks according to job requirements. |
|  |  | 2.2 | Position safety signage, barriers, containment and engineering controls and confirm that area is safe to access and work in according to WHS and job requirements. |
|  |  | 2.3 | Select the required odour counteractants and appropriate equipment and check for correct operation for safe use according to manufacturer instructions. |
|  |  | 2.4 | Determine sequence of tasks for maximum hygiene and efficiency of odour removal. |
| 3 | Apply odour control treatment. | 3.1 | Pre-clean by removing as much of the source of the odour as possible, including unsalvageable debris that contributes to odour generation and recontamination of cleaned and deodorised areas, according to WHS and job requirements. |
|  |  | 3.2 | Clean salvageable surfaces and items in order to physically remove odour-causing residue according to WHS and job requirements. |
|  |  | 3.3 | Seal salvageable surfaces to encapsulate odour and prevent progressive recontamination according to WHS and job requirements. |
|  |  | 3.4 | Treat remaining odour with an odour counteractant according to manufacturer instructions and WHS requirements. |
|  |  | 3.5 | Create a deodorising fog or gas that seeks out and combines with smoke-related odour-causing substances according to manufacturer instructions and job requirements. |
|  |  | 3.6 | Assess outcome of odour removal activities and repeat action when odour has not been removed sufficiently to meet job requirements. |
| 4 | Complete work activities. | 4.1 | Record odour treatment and outcomes and seek independent verification or sign-off, as required, according to job and WHS requirements. |
|  |  | 4.2 | Remove safety signage barriers, containment and engineering controls and dispose of soil and waste according to WHS and environmental requirements. |
|  |  | 4.3 | Clean and/or dispose of PPE as required by organisational standards. |
|  |  | 4.4 | Check equipment for faults, wear or damage and report according to organisational requirements. |
|  |  | 4.5 | Store equipment and materials according to organisational requirements. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is not equivalent to CPPCLO3010 Apply odour control to carpets.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b

Assessment Requirements for CPPCLO4105 Apply odour control to structures and contents

# Modification History

| Release 1 | This version first released with CPP Property Services Training Package Release 15.0. Supersedes and is not equivalent to CPPCLO3010 Apply odour control to carpets. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the elements and performance criteria of this unit by:

applying odour control to remove four of the following:

urine odour from both natural fibre and synthetic carpet

animal or human waste odour from upholstery

mould odour from plasterboard

odour from decaying matter from built-in cabinetry

fire damage odour from timber furniture

chemical odour from furnishings.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

aspects of the following relevant to removal of odours from structures and contents:

work health and safety (WHS) requirements

Safe Work Australia regulations and guidelines

job safety analyses (JSAs) and safe work method statements (SWMS)

manufacturer instructions and safety data sheets (SDS)

IICRC S700 Standard for Professional Fire and Smoke Damage Restoration or Australian equivalent

AS/NZS 3733 Textile floor coverings – Cleaning maintenance of residential and commercial carpeting

Environment Protection Authority (EPA) and local requirements for prescribed waste disposal

personal and site security procedures and requirements:

site access - security cards, codes and keys

areas of the worksite not to be accessed

alarm activation and deactivation

maintaining client privacy and confidentiality

contingencies if personal safety at risk

building system considerations relevant to odour control:

structural components - methods of construction and materials

heating, cooling and ventilation, including heating, ventilation and air conditioning (HVAC) systems

potential locations of not readily observable odour producing matter - behind and under plaster walls, ceilings, pipes, appliances, insulation, panelling, wallpaper, floors, flooring, carpet, cabinetry and interstitial spaces

unintended consequential damage arising from event and mitigation

types of hazards and risks and ways to control:

types of personal protective equipment (PPE) and correct selection, care, use and disposal

slips, trips, falls and spills

manual-handling techniques when carrying, lifting, pushing and pulling

identification and safe practices with asbestos on site

correct use of signage, barriers, containment and engineering controls and PPE

safe use of electrical equipment

cleaning principles:

cleaning agent, time, agitation and temperature

significance of pH value of cleaning agents

sequence and direction of cleaning

achieve results while doing no harm

factors determining restorability:

type of odour and duration of exposure

material composition

cost of restoration

cost of replacement

sentimental, legal, artistic, cultural and historical value

potential secondary harm and waste created by restoration methods

situations and procedure for seeking specialist advice:

work requiring insurance company approval

situations requiring waiver

specialised contents such as artwork

high value and irreplaceable items

role, strengths and limitations of specialists who may provide advice:

Indoor Environmental Professional (IEP)

occupational hygienist

building professional

insurance representative

microbiologist

origins of odour - cooking, animal, rotting/putrid, burnt/smoky, chemical/solvent and vegetable

types of hazardous odours and necessary precautions

methods and precautions for odour control on surface types:

porous

semi-porous

non-porous

textiles

functions and features of odour control equipment, safe use, routine maintenance and storage:

air scrubbers

carpet groomers

chemical hand spray units

foggers

ozone machines

steam generators

types of odour control agents - their purpose, action, correct use, precautions, safe handling, storage and disposal:

gas phase type processes

ozone

hydroxyls

cold plasma

odour control methods

environmentally sustainable work practices and compliance with environmental regulations:

ways of minimising negative environmental impact

conserving energy and water

recycling

waste disposal of used materials and restoration agents

disposal of prescribed waste.

# Assessment Conditions

Assessors must satisfy the requirements for assessors contained in the Standards for Registered Training Organisations.

Competency is to be assessed in the workplace or a simulated environment that accurately reflects performance in a real workplace setting where these skills and knowledge would be performed.

Candidates must have access to:

client work order, instructions or service agreement

organisational documentation, policies and procedures.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b