# CPPFES2003 Safely move loads and dangerous goods

Release: 1

CPPFES2003 Safely move loads and dangerous goods

# Modification History

| Release 1 | This version first released with CPP Property Services Training Package Release 13.0. Supersedes and is equivalent to CPPFES2003A Safely move loads and dangerous goods. Unit updated to meet the Standards for Training Packages 2012. Updated licensing statement. |
| --- | --- |

# Application

This unit of competency specifies the skills and knowledge required to safely move materials, loads and hazardous or dangerous goods used in the fire protection industry in such a way as to avoid injury to self and others, and prevent damage to product, property and the environment. It includes handling, storing and transporting containers that may contain ozone depleting substances (ODS) and synthetic greenhouse gases (SGG).

The unit is suitable for those with basic skills and knowledge undertaking routine work tasks under the direction of more experienced workers.

Candidates must hold a current, nationally recognised licence required to operate vehicles when transporting loads or dangerous goods by road.

ODS and SGG are gaseous fire-extinguishing agents listed in Schedule 1 of the Ozone Protection and Synthetic Greenhouse Gas Management Act 1989 and by law, can only be handled by people who hold an appropriate extinguishing agent handling licence (EAHL). This unit supports one or more EAHLs prescribed under the Act. For further information, check with the relevant regulatory authority.

# Pre-requisite Unit

Nil.

# Unit Sector

Fire Protection Inspection and Testing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare to move loads and dangerous goods | 1.1 | Read work instructions to identify requirements for moving loads and dangerous goods and clarify work tasks with relevant persons. |
|  |  | 1.2 | Read safety signs, safety data sheets (SDS) and relevant dangerous goods documentation to identify types of gases to be handled, stored or transported, and required safe handling practices. |
|  |  | 1.3 | Plan work activities to comply with relevant dangerous goods regulations, Australian Standards, work health and safety (WHS) and workplace requirements. |
|  |  | 1.4 | Assess dangerous goods container sizes and movement paths to identify hazards. |
|  |  | 1.5 | Assess risks posed by identified hazards and apply required risk control methods according to workplace requirements. |
|  |  | 1.6 | Select and use required personal protective equipment (PPE). |
| 2 | Use manual-handling techniques to move loads and materials. | 2.1 | Select manual-handling techniques appropriate to the type of loads and materials being moved. |
|  |  | 2.2 | Use selected manual-handling techniques to safely move loads and materials according to workplace requirements |
| 3 | Use mechanical-handling aids to move loads and materials. | 3.1 | Select mechanical-handling aids appropriate to the type of loads and materials being moved. |
|  |  | 3.2 | Use selected mechanical-handling aids to safely move loads and materials according to workplace requirements. |
|  |  | 3.3 | Inspect and maintain mechanical-handling aids to ensure correct working order according to manufacturer requirements. |
|  |  | 3.4 | Report faulty mechanical-handling aids according to workplace requirements. |
| 4 | Adhere to regulatory requirements for moving and transporting hazardous or dangerous materials. | 4.1 | Move hazardous or dangerous materials in compliance with regulatory and workplace requirements for ODS, SGG and dangerous goods. |
|  |  | 4.2 | Obtain and use transport documents and signage for hazardous or dangerous materials to be transported by road to comply with regulatory requirements. |
|  |  | 4.3 | Comply with regulatory and workplace requirements for ODS, SGG and dangerous goods to transport hazardous or dangerous materials by road. |
|  |  | 4.4 | Store hazardous or dangerous materials that form part of fire protection equipment or systems according to workplace and regulatory requirements. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPPFES2003A Safely move loads and dangerous goods.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b

Assessment Requirements for CPPFES2003 Safely move loads and dangerous goods

# Modification History

| Release 1 | This version first released with CPP Property Services Training Package Release 13.0. Supersedes and is equivalent to CPPFES2003A Safely move loads and dangerous goods. Unit updated to meet the Standards for Training Packages 2012. Updated licensing statement. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the elements and performance criteria of this unit by safely moving loads and dangerous goods used in the fire protection industry, including:

manually handling at least one class 2.2 container having a minimum water capacity volume of 80 litres from its secured, stored position onto a mechanical-handling aid, typically a trolley fitted with restraining straps, and then relocating the cylinder at least three metres from that position, and removing it from the mechanical-handling aid to a secured location

safely transporting two gaseous containers using a vehicle in compliance with dangerous goods regulations.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

dangerous goods commonly used in the fire protection industry and key requirements, including:

general storage requirements for all dangerous goods

identification of dangerous goods by diamond-shaped symbol

requirements for transporting dangerous goods by a vehicle, especially class 2.2 for placarded loads and bulk loads

storage requirements for gaseous containers, especially those containing ozone depleting substances (ODS) and synthetic greenhouse gases (SGG)

extinguishing agent handling licence (EAHL) requirements

handling and storage requirements for scheduled ODS and SGG in accordance with the Fire Protection Industry (ODS &SGG) Board ODS & SGG Good Practice Guide

manual-handling techniques, including implications of:

ignoring safety precautions used with mechanical manual-handling aids

incorrect manual-lifting techniques

maximum safe loads for vehicles

meaning of material handling applied to the role of a service technician

methods used to prevent ODS and SGG emissions in the workplace

purpose and structure of relevant dangerous goods legislation, including HAZCHEM signs and emergency information panels

purpose of hazard identification, hazard assessment and hazard control related to moving loads and dangerous goods

reasons for preventing ODS and SGG emissions in the workplace

workplace requirements for moving loads and dangerous goods:

documentation

environmental protection

maintenance of tools and equipment

work health and safety (WHS), including hazard and risk identification and control.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to documentation, tools, equipment, loads, materials and vehicle required to achieve the performance evidence.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b