# Assessment Requirements for CPPHES5001 Conduct thermal performance assessment of complex residential buildings

Release: 2

Assessment Requirements for CPPHES5001 Conduct thermal performance assessment of complex residential buildings

# Modification History

| Release 2 | This version first released with CPP Property Services Training Package Release 14.0. Updated reference to reflect correction in Modification History from CPC Property Services 5.0 to CPP Property Services 9.0. |
| --- | --- |
| Release 1 | This version first released with CPC Property Services Training Package Release 5.0. Supersedes and equivalent to CPPHSA5001A Assess thermal performance of complex residential buildings. Updated to meet the Standards for Training Packages. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by using an accredited Nationwide House Energy Rating Scheme (NatHERS) software tool to conduct thermal performance assessments of three complex residential buildings. The buildings assessed must include different designs:

appropriate to a tropical, a temperate and a cold climate zone

to fit sites with different exposures, topography and a wide diurnal temperate range

that use climate-appropriate construction methods and materials

involve complex levels, built shading devices and void arrangements and incorporate the following:

one single storey dwelling with:

a minimum of three bedrooms

one ground floor split level

two different ground floor construction

three different roof constructions including raked ceilings, sloping walls and at least three clerestory windows, dormer construction, split style walls

one triple storey dwelling (a minimum of four bedrooms) with:

an underground basement/garage with a different footprint

a ground floor and a first floor

a habitable attic space with at least two dormer windows

a lift from the basement to both the ground floor and first floor

one open-plan studio apartment.

The assessments must be conducted according to the requirements of the software, technical notes and jurisdictional regulations.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

Australian climate zones, characteristics and data used in thermal performance assessments

design features that influence the thermal comfort of buildings

energy units of measurement and terminology associated with thermal performance assessments:

energy efficiency

heating and cooling loads

solar heat gain coefficient (SHGC) and U-value

R-values

options to improve thermal performance of buildings:

construction materials

sealing the building envelope

floor coverings

insulation levels

orientation

overshadowing

passive heating and cooling

shade

structural changes

thermal mass

ventilation

window size, location, type and coverings

principles of passive design:

glazing

insulation

orientation

passive cooling

passive solar heating

shading

thermal bridging and breaks

regulatory requirements for thermal performance assessments of residential buildings categorised as Class 1, 2, 4 and 10a of the National Construction Code (NCC)

thermal performance properties of a variety of building materials:

solar absorptance

heat transfer coefficients and U-values

material thickness

reflectivity

resistance heat flow up and down

solar heat gain coefficient and emissivity

thermal bridging and breaks

thermal resistance and R-values

thermal performance ratings:

current rating requirements

documentation of ratings

legal requirements

uses

types and operation of NatHERS accredited software tools:

assumptions and limitations as to what can be assessed inherent in NatHERS software protocols and accredited software

building and external elements included in NatHERS thermal performance assessments

requirements for modelling complex buildings and non-standard materials.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to:

current accredited NatHERS software tool and associated equipment and manuals

the NCC and jurisdictional guidelines associated with conducting NatHERS assessments

residential building design documentation and technical information to allow achievement of the performance evidence

NatHERS technical notes and software accreditation protocol.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b