# Assessment Requirements for CPPPMT3017 Maintain, service and repair pest management equipment

Release: 1

Assessment Requirements for CPPPMT3017 Maintain, service and repair pest management equipment

# Modification History

Release 1.

Replaces superseded equivalent CPPPMT3017A Repair and maintain service equipment.

This version first released with CPP Property Services Training Package Version 2.

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit, and must develop and implement an equipment maintenance plan for the delivery of pest management services, including daily, weekly and monthly maintenance tasks for equipment as listed below.

In doing the above, the person must also:

conduct three of the following regular maintenance tasks:

first aid kits – check and replenish weekly or after use

nozzles – clean monthly

pneumatic hand sprayers – clean and lubricate monthly

respirators – clean and replace filters weekly at the minimum

spill kits – check and replenish monthly or after use

spray guns – clean and lubricate weekly

conduct three of the following servicing and repair tasks:

nozzles – replace ‘O’ ring (cone jet) as required

pneumatic hand sprayers – fit repair kit to pneumatic sprayer annually

respirators – repair exhalation valve and head strap as required

simple centrifugal pump – fit seal kit

spray guns – fit seal kit to spray gun.

When implementing the maintenance plan and conducting the above servicing and repairs a person must also:

comply with company and compliance requirements

identify hazards and control risks associated with maintenance, servicing and repair of pest management equipment

apply safe work methods for:

manual handling

operating equipment

select equipment and materials suitable for the tasks.

# Knowledge Evidence

A person demonstrating competency in this unit must demonstrate knowledge of:

chemicals’ routes of entry into human body and potential symptoms of exposure

compliance requirements relevant to maintaining, servicing and repairing pest management equipment

company procedures relevant to maintaining, servicing and repairing pest management equipment, including:

documentation requirements

emergency response procedures

enterprise maintenance and servicing policies and procedures

health and safety procedures, including hierarchy of hazard control

injury, dangerous occurrence and incident reporting

maintenance schedules

manual-handling techniques

work documentation and reporting procedures

hazards and risks of storing, preparing, using and disposing of chemicals

product knowledge, including manufacturer specifications for equipment and products being used during performance evidence tasks

sources of information about spare parts availability and supply

types of pest management plant and equipment and their functions.

# Assessment Conditions

Suitable assessment of performance requires:

equipment:

pest management equipment requiring regular maintenance as listed in the performance evidence

pest management equipment requiring servicing, as listed in the performance evidence

pest management equipment requiring repair, as listed in the performance evidence

equipment for conducting maintenance, servicing and repairs, including personal protective equipment (PPE)

specifications:

company policies and procedures

equipment manuals and maintenance schedules

manufacturer specifications

plain English guides to legislative, health and safety and environmental requirements

physical conditions:

suitable workshop for servicing and repairing pest management equipment

relationships with team members and supervisor:

work may be conducted individually or as part of a team

timeframe:

according to work order.

### Assessor requirements

As a minimum, assessors must satisfy the assessor requirements in the Standards for Registered Training Organisations (RTOs) current at the time of assessment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b