# CPPUPM3006 Manage pests by applying pesticides

Release: 3

CPPUPM3006 Manage pests by applying pesticides

# Modification History

| Release 3 | This version first released with CPP Property Services Training Package Release 14.0. Typographical correction from pscoids to psocids in the Knowledge Evidence and Performance Evidence. |
| --- | --- |
| Release 2 | This version first released with CPP Property Services Training Package Release 13.1. Fixed incorrect terminology replaced, paperlice reference with pscoids in Performance Evidence and Knowledge Evidence. |
| Release 1 | This version first released with CPP Property Services Training Package Release 9.0. Supersedes and equivalent CPPPMT3006 Manage pests by applying pesticides. |

# Application

This unit specifies the skills and knowledge required to apply pesticides to manage a range of non-timber pests such as ants, cockroaches, rodents and spiders in premises where public health and safety and the amenity of the community and environment may be affected.  The unit includes gaining a thorough understanding of client requirements and factors contributing to the pest problem and assessing the nature and extent of pest activity at the site.  It requires selecting appropriate equipment, pesticides and application methods to safely and efficiently manage pests.

The unit does not apply to inspecting for, or managing termites or other timber pests, or non-chemical pest management where the environment is modified to manage pests.

A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without close supervision.

This unit forms part of the licensing requirements for people engaged in pest management in some states and territories. For further information, check with the relevant regulatory authority.

# Pre-requisite Unit

Nil

# Unit Sector

Urban Pest Management

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Confirm client and compliance requirements. | 1.1 | Review work instructions to confirm client requirements, site location and pest management timeframes and clarify issues with relevant persons. |
|  |  | 1.2 | Review workplace policies and procedures and regulatory and industry requirements for managing pests using pesticides and apply to work instructions to ensure compliance. |
| 2 | Inspect work site and identify pests or pest activity. | 2.1 | Establish contact with relevant persons and access work site according to client and workplace requirements. |
|  |  | 2.2 | Determine inspection method and select and use required inspection tools and equipment including personal protective equipment (PPE). |
|  |  | 2.3 | Identify hazards associated with inspection tasks and assess and control risks at the site according to workplace requirements. |
|  |  | 2.4 | Examine site buildings and surrounds to identify pest entry points, evidence of activity and risk factors according to work instructions. |
|  |  | 2.5 | Assess evidence to identify pest types and extent and potential for pest activity and record findings. |
| 3 | Plan pest management activities. | 3.1 | Select pesticide and application methods to manage identified pest types and extent and potential of pest activity. |
|  |  | 3.2 | Conduct risk assessment and plan risk management strategies to ensure safety and regulatory compliance when managing pests using pesticides. |
|  |  | 3.3 | Identify factors affecting completion of pest management activities or compliance and obtain required information or assistance. |
|  |  | 3.4 | Prepare and document a pest management plan that addresses the client’s pest problem and meets workplace and compliance requirements. |
|  |  | 3.5 | Discuss proposed pest management methods with client to clarify issues and concerns and confirm understanding of chemical control aspects of pest management plan and obtain approval to proceed. |
|  |  | 3.6 | Calculate work site size and usage pattern to plan work activities to ensure safety of personnel and efficient use of equipment and pesticides. |
| 4 | Prepare equipment and pesticides and treat work site. | 4.1 | Install signs or barriers to maximise protection of public health and safety during site treatment, notify occupants, and implement other risk control measures required to manage hazards identified at the work site. |
|  |  | 4.2 | Read and follow manufacturers’ instructions and safety data sheets (SDS) for selected pesticides and application methods. |
|  |  | 4.3 | Select and use PPE for pesticide application according to manufacturers’ instructions and SDS. |
|  |  | 4.4 | Select pesticides according to risk assessment and pest management plan. |
|  |  | 4.5 | Select, check and adjust application equipment and confirm correct and safe operation according to manufacturers’ instructions and workplace requirements. |
|  |  | 4.6 | Calculate application area, estimate required quantities of pesticides, and prepare pesticides for application following manufacturers’ instructions and directions on product labels. |
|  |  | 4.7 | Calibrate pump sprayer and check nozzles and flow rate to ensure correct application for surface being treated according to manufacturers’ instructions. |
|  |  | 4.8 | Apply pesticides at work site and check to ensure correct application according to pest management plan and compliance requirements. |
|  |  | 4.9 | Recognise and promptly respond to emergency situation according to workplace requirements. |
| 5 | Finalise work and complete documentation. | 5.1 | Remove equipment, pesticides and waste from pest management activities and restore work site to meet client and compliance requirements. |
|  |  | 5.2 | Transport and store pesticides and dispose of waste removed from work site according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 5.3 | Eliminate hazards prior to site handover and remove signage or barriers according to workplace and compliance requirements. |
|  |  | 5.4 | Notify relevant persons of residual hazards, necessary hazard reduction measures and site re-entry periods. |
|  |  | 5.5 | Clean and decontaminate equipment and PPE according to manufacturers’ instructions and workplace requirements. |
|  |  | 5.6 | Check equipment and PPE to confirm safe and correct operation, record required maintenance and store to allow ready access and re-use. |
|  |  | 5.7 | Treat exposed skin to remove any contamination according to product manufacturers’ instructions and workplace requirements. |
|  |  | 5.8 | Complete and distribute pest management documentation to meet workplace and compliance requirements. |

# Foundation Skills

Candidates require:

oral communication skills to use clear explanations, active listening and questioning skills to convey, clarify and confirm information provided to clients

reading skills to interpret key requirements of manufacturers’ instructions and SDS

numeracy skills to dilute and mix pesticides in the correct ratios.

# Unit Mapping Information

Supersedes and equivalent to CPPPMT3006 Manage pests by applying pesticides.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b

Assessment Requirements for CPPUPM3006 Manage pests by applying pesticides

# Modification History

| Release 3 | This version first released with CPP Property Services Training Package Release 14.0. Typographical correction from pscoids to psocids in the Knowledge Evidence and Performance Evidence. |
| --- | --- |
| Release 2 | This version first released with CPP Property Services Training Package Release 13.1. Fixed incorrect terminology replaced, paperlice reference with pscoids in Performance Evidence and Knowledge Evidence. |
| Release 1 | This version first released with CPP Property Services Training Package Release 9.0. Supersedes and equivalent CPPPMT3006 Manage pests by applying pesticides |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by selecting and applying pesticides internally, and externally as required, in order to manage non-timber pests in ten different residential and/or commercial premises.

The following different types of pests must be managed during work across the ten premises:

ants

cockroaches

rodents

spiders

a minimum of four of the following:

bed bugs

bees and wasps

fabric pests

fleas

flies

mites

occasional invaders: amphipods, millipedes, springtails, thrips

psocids

stored product pests.

Work across the ten premises must involve using:

bait stations

electrical dust applicator

hand puffer

pump up sprayer

insecticides and rodenticides, with application methods involving:

applying pesticide liquids, powders, gels and granules

installing and securing baits

space treatments that involve preparation, no-entry notification, using a fogger or mister, ventilation and re-entry processes.

Pest management plans for each premises must detail:

identified pest types and potential for pest activity

extent of pest activity or infestation including the source, scope and degree of pest activity and contributing risk factors

pest management options

types and quantities of chemicals to be used

chemical application methods

risk management plan

advice on health, safety, environmental protection and other compliance and mandatory notification requirements

advice on pest prevention strategies

follow-up pest management advice, monitoring and call-back schedule.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

chemicals’ routes of entry into the human body and potential symptoms of exposure

compliance requirements associated with managing pests by applying pesticides:

Australian standards, quality assurance and certification requirements

industry guides, advisory standards and relevant industry codes of practice including A Code of Practice for the Control of Bed Bug Infestations in Australia

environmental requirements:

clean up, containment or isolation of hazards

emergency chemical spill control measures

hazardous materials handling

waste handling and disposal

key requirements of commonwealth, state and territory legislation and local government regulations:

chemical use, registers and manifests

dangerous goods

environmental protection

licensing for pest management

mandatory notifications

privacy

public health and safety:

licence endorsements and conditions or limitations

occupant notifications

record of site conditions

warning signs

ecology, biology, lifecycles, habits and harbourages of the following types of pests:

ants

bed bugs

bees and wasps

cockroaches

fabric pests (at least three different species in the geographical area)

fleas

flies (at least three different species in the geographical area)

mites

occasional invaders:  springtails, thrips, amphipods and millipedes

psocids

rodents

spiders

stored product pests (at least three different species in the geographical area)

humane euthanasia procedures used in pest management

information and industry terminology typically contained in pest management work instructions

meaning of integrated pest management

pesticide controls for managing pests in a range of environments, conditions, industries and premises

potential effects of pesticides on public health and the environment and the difference between primary and secondary poisoning associated with rodent baits

purpose of, and information typically contained in safety data sheets (SDS)

signs of termite activity and implications of not reporting suspected termite activity for follow-up action

tools, equipment and materials used when managing pests by applying pesticides:

inspection tools and equipment:

camera

ladder

magnifying glass

mirror

portable lighting or torch

probe

specimen bottles

personal protective equipment (PPE):

eye, ear and respiratory protection

first aid kit

site-specific PPE and clothing

soap and towel

sun protection

pesticide application:

bait stations

dust applicators

hand puffers

hot and cold foggers for space spraying

misters for surface spraying

pump up, hand-held and backpack sprayers

truck-mounted pump and hose rigs

types of pesticides used to manage non-timber pests and methods for safe handling and application:

chemical group, target site and mode of action

surfaces, substrates and suitable formulation types

application and spray patterns

typical hazards encountered when managing pests by applying pesticides, and risk control measures for each:

animal bites and stings

chemicals and residual pesticides

confined or restricted spaces

electrical

environmental contamination

fire

guard dogs

hazards affecting native species

ladder use

lighting

manual handling

moving vehicles and machinery

noise

remote locations

respiratory

slips and trips

temperature

ultraviolet radiation

work at heights

workplace policies and procedures that apply to managing pests by applying pesticides:

documentation and reporting

equipment maintenance and servicing

format and content requirements for pest management plans

site arrival, sign-in and pre-work preparations and authorities

work health and safety:

emergency response procedures

hazard identification, risk assessment and control using the hierarchy of control

injury, dangerous occurrence and incident reporting

safe work practices including manual handling and ladder use

signs and barriers required to be installed to protect public health and safety during pest management work.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Evidence of assessment for pest management activities in seven of the ten premises as specified in the performance evidence, must be based on documentation and photographic evidence that has been validated by a workplace supervisor.  A workplace supervisor is a person duly authorised to perform and validate the work being undertaken.

Evidence of assessment for pest management activities in three of the ten premises as specified in the performance evidence:

must be based on direct observation and assessment of the candidate by an assessor, physically or via audio and visual media

must involve managing ants, cockroaches, rodents and spiders

must be in a real workplace for a minimum of two premises

may be simulated for one premises provided the simulation allows demonstration of all performance criteria.

Candidates must have access to premises and associated work instructions, information, SDS, materials, tools, equipment and PPE required to achieve the performance evidence, including relevant industry codes of practice including A Code of Practice for the Control of Bed Bug Infestations in Australia.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b