# CPPUPM3011 Manage organisms by applying fumigants to commodities and environments

Release: 2

CPPUPM3011 Manage organisms by applying fumigants to commodities and environments

# Modification History

| Release 2 | This version first released with CPP Property Services Training Package Release 10.3 Reference to AS 2476 has been removed from the Knowledge and Assessment Conditions. |
| --- | --- |
| Release 1 | This version first released with CPP Property Services Training Package Release 9.0. Supersedes and equivalent CPPPMT3011 Manage organisms by applying fumigants to commodities and environments |

# Application

This unit specifies the skills and knowledge required to apply fumigants to control organisms and organism activity that affect the health, safety or amenity of persons, commodities or environments in domestic, commercial, industrial and maritime situations.  The unit requires understanding client requirements, assessing the site, and selecting and using equipment, fumigants and application methods to effectively manage organisms.

Work is conducted by teams of at least two people in compliance with state and territory health and safety legislation.  A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without close supervision.

Fumigants are most frequently applied by pest technicians to meet quarantine conditions for exported or imported goods but are also applied in commercial grain handling and storage environments.

This unit does not apply to horticultural or agricultural situations involving crop growing.

This unit forms part of the licensing requirements for people engaged in pest management in some states and territories. For further information, check with the relevant regulatory authority.  Where required, the fumigator must meet the licensing requirements of the relevant Australian or international quarantine authority.

# Pre-requisite Unit

Nil

# Unit Sector

Urban Pest Management

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Confirm work and compliance requirements. | 1.1 | Review work instructions or quarantine order to confirm fumigation requirements and discuss client needs and expectations with relevant persons. |
|  |  | 1.2 | Confirm authority of client requesting fumigation services and clarify fumigation site and target in consultation with relevant persons. |
|  |  | 1.3 | Review workplace policies and procedures and regulatory and industry requirements for organism fumigation and apply to work instructions to ensure compliance. |
| 2 | Access and assess work site. | 2.1 | Select and fit required personal protective equipment (PPE) according to workplace requirements. |
|  |  | 2.2 | Access work site, identify hazards and control risks according to workplace requirements. |
|  |  | 2.3 | Assess work site to identify access points and security arrangements and characteristics impacting fumigation according to workplace requirements. |
|  |  | 2.4 | Determine site size and usage patterns to ensure safety of personnel and efficient use of fumigants and equipment during fumigation. |
| 3 | Determine organism types and fumigation method. | 3.1 | Confirm types of organisms and commodities present at the work site according to workplace requirements. |
|  |  | 3.2 | Select fumigation method and fumigant to suit types of organisms, work site environment and commodities present according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 3.3 | Establish capacity to create fumigation conditions at work site and promptly report factors affecting completion of work instructions or quarantine order to relevant persons. |
|  |  | 3.4 | Advise relevant persons of risks associated with fumigation including non-target contamination and damage |
| 4 | Prepare for fumigation. | 4.1 | Install signs or barriers to maximise protection of public health and safety during fumigation and implement other risk control measures required to manage hazards at the work site. |
|  |  | 4.2 | Notify personnel in adjoining properties and required authorities of proposed fumigation according to workplace and compliance requirements. |
|  |  | 4.3 | Implement security measures to control unauthorised access to work site during fumigation and prescribed non re-entry periods according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 4.4 | Create conditions to ensure that essential fumigant concentrations are retained for required period within fumigation enclosure according to manufacturers’ instructions. |
|  |  | 4.5 | Select and use PPE for fumigant application according to manufacturers’ instructions and safety data sheet (SDS) for fumigant type and workplace requirements. |
|  |  | 4.6 | Select and calibrate application equipment and confirm correct and safe operation according to manufacturers’ instructions and workplace requirements. |
|  |  | 4.7 | Access fumigant and transport to application site according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 4.8 | Calculate and prepare fumigant dosages according to fumigant type, fumigation target and enclosure volume and temperature following manufacturers’ instructions and compliance requirements. |
| 5 | Apply fumigant and monitor fumigation. | 5.1 | Treat fumigation enclosure using equipment, fumigant and fumigation methods according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 5.2 | Check and adjust fumigation enclosure seal to ensure optimal application according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 5.3 | Assess fumigant escape throughout fumigation exposure period to control impact on public health and safety according to workplace and compliance requirements. |
|  |  | 5.4 | Secure and continuously monitor fumigation site to prevent unauthorised access and maintain safe conditions according to workplace and compliance requirements. |
|  |  | 5.5 | Monitor and maintain integrity of enclosure seal and fumigant concentrations over fumigation exposure period according to manufacturers’ instructions. |
|  |  | 5.6 | Recognise and promptly respond to emergency situation according to workplace requirements. |
| 6 | Ventilate fumigation enclosure. | 6.1 | Establish suitable conditions for enclosure ventilation to control risks to public health and safety according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 6.2 | Apply approved method to safely release fumigant and ventilate enclosure to remove fumigant in a safe and controlled manner according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 6.3 | Aerate fumigation enclosure and commodities present according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 6.4 | Test fumigation enclosure and commodities present for evidence of residual fumigant according to manufacturers’ instructions, workplace and compliance requirements. |
| 7 | Finalise work and complete documentation. | 7.1 | Remove equipment, fumigant and waste and restore work site to meet work instructions or quarantine order and compliance requirements. |
|  |  | 7.2 | Eliminate hazards prior to site handover and remove signage or barriers according to workplace and compliance requirements. |
|  |  | 7.3 | Notify relevant persons of residual hazards, necessary risk control measures and site re-entry periods. |
|  |  | 7.4 | Clean and decontaminate equipment and PPE according to manufacturers’ instructions and workplace requirements. |
|  |  | 7.5 | Check equipment and PPE to confirm safe and correct operation, record required maintenance and store to allow ready access and re-use. |
|  |  | 7.6 | Transport and store fumigant according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 7.7 | Dispose of waste from fumigation activities according to work instructions or quarantine order, workplace and compliance requirements. |
|  |  | 7.8 | Treat exposed skin to remove any contamination according to SDS and workplace requirements. |
|  |  | 7.9 | Complete and distribute required documentation and update client records according to workplace and compliance requirements. |

# Foundation Skills

Candidates require:

oral communication skills to use questioning and active listening when discussing client needs and expectations

reading skills to interpret key requirements of work instructions, manufacturers’ instructions and SDS

numeracy skills to:

calculate required fumigant concentrations at various stages of the fumigation period

calculate percentage variations in fumigant concentrations

writing skills to accurately and legibly complete workplace and compliance documentation.

# Unit Mapping Information

Supersedes and equivalent to CPPPMT3011 Manage organisms by applying fumigants to commodities and environments.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b

Assessment Requirements for CPPUPM3011 Manage organisms by applying fumigants to commodities and environments

# Modification History

| Release 2 | This version first released with CPP Property Services Training Package Release 10.3 Reference to AS 2476 has been removed from the Knowledge and Assessment Conditions. |
| --- | --- |
| Release 1 | This version first released with CPP Property Services Training Package Release 9.0. Supersedes and equivalent CPPPMT3011 Manage organisms by applying fumigants to commodities and environments |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by fumigating five different sites from the types listed below, according to the type of licence held:

aircraft

building

burrow

chamber

container

sheeted stack

ship

silo

soil.

Fumigation must target one or more of the following organisms:

arachnids

bacteria

fungi

insects

molluscs

moulds

plants

rabbits

rodents

seeds.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

chemicals’ routes of entry into the human body and potential symptoms of exposure

commodity absorption and desorption rates

compliance requirements associated with managing organisms by applying fumigants to commodities and environments in domestic, commercial, industrial and maritime situations:

environmental requirements:

clean up, containment or isolation of hazards

emergency chemical spill control measures

hazardous materials handling

regulations, by-laws and guidelines of environmental protection agencies and government departments

waste handling and disposal

industry guides, advisory standards and relevant industry codes of practice

key requirements of commonwealth, state and territory legislation and local government regulations:

chemical use, registers and manifests

consumer protection

dangerous goods

fumigation regulations and licence requirements

mandatory notifications

privacy

public health and safety

quarantine requirements

work health and safety (WHS)

public health and safety:

licence endorsements and conditions or limitations

occupant notifications

record of site conditions

warning signs

equipment used when applying fumigants:

fumigant application equipment and calibration methods

personal protective equipment (PPE):

eye, ear and respiratory protection

first aid kit

site-specific PPE and clothing

soap and towel

sun protection

fumigation methods and associated:

fumigation conditions, site and enclosure requirements

organisms eradicated through fumigation

product knowledge including manufacturers’ instructions for equipment and products being used

risks of effect of fumigant on public health and environment

information and industry terminology typically contained in work instructions and quarantine orders

purpose of, and information typically contained in safety data sheets (SDS)

test methods and readings for monitoring fumigant concentrations

types of fumigants used to manage organisms in commodities and environments:

application methods

environmental conditions such as withholding periods

preparation requirements

safety requirements for handling, use and decontamination

types of organisms treated

typical hazards encountered when managing organisms by applying fumigants to commodities and environments, and risk control measures for each:

animal bites and stings

chemicals

confined or restricted spaces

electrical

environmental contamination

fire

lighting

manual handling

moving vehicles and machinery

noise

remote locations

respiratory

slips and trips

temperature

ultraviolet radiation

work at heights

workplace policies and procedures that apply to managing organisms by applying fumigants:

documentation and reporting

WHS:

emergency response procedures

hazard identification, risk assessment and control using the hierarchy of control

injury, dangerous occurrence and incident reporting

safe work methods.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Fumigations must be conducted with at least one other person and supervised by a licensed fumigator.  Of the five required simulations, two must be conducted in a real workplace and three may be simulated

Candidates must have access to fumigation sites and associated work instructions, information, SDS, fumigants, tools, equipment and PPE required to achieve the performance evidence including relevant industry codes of practice.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b