# CPPUPM3018 Maintain equipment and pesticide storage area in pest management vehicles

Release: 1

CPPUPM3018 Maintain equipment and pesticide storage area in pest management vehicles

# Modification History

| Release 1 | This version first released with CPP Property Services Training Package Release 9.0. Supersedes and equivalent CPPPMT3018 Maintain equipment and pesticide storage area in pest management vehicles |
| --- | --- |

# Application

This unit specifies the skills and knowledge required to set up, stock and control a storage area for equipment and pesticides within a pest management vehicle.  It includes monitoring and controlling the security, labelling, movement and storage of equipment and pesticides and ensuring strict compliance with workplace and regulatory requirements.  The unit requires maintaining the pest management vehicle to ensure it is clean, safe and roadworthy in readiness for pest management work.

A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without close supervision.

This unit forms part of the licensing requirements for people engaged in pest management in some states and territories. For further information, check with the relevant regulatory authority.

# Pre-requisite Unit

Nil

# Unit Sector

Urban Pest Management

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Confirm compliance requirements and set up vehicle storage area. | 1.1 | Review workplace policies and procedures and regulatory and industry requirements for equipment and pesticide storage in pest management vehicles and apply to work instructions to ensure compliance. |
|  |  | 1.2 | Check pest management vehicle capacities against intended use and verify compliance with regulatory requirements. |
|  |  | 1.3 | Secure storage area from unauthorised access according to regulatory requirements. |
|  |  | 1.4 | Identify existing hazards associated with vehicle and pesticides and implement required risk control measures. |
|  |  | 1.5 | Organise storage area and separate equipment and passenger compartments to minimise transportation risks. |
|  |  | 1.6 | Equip vehicle to enable emergency response to be implemented according to workplace and compliance requirements. |
| 2 | Equip vehicle storage area. | 2.1 | Select, check for correct operation, and use personal protective equipment (PPE) according to safety data sheets (SDS) and manufacturers’ instructions for chemicals to be stored. |
|  |  | 2.2 | Check chemical containers to verify suitable condition for storage and transport according to compliance requirements. |
|  |  | 2.3 | Identify chemical storage specifications and store chemicals in vehicle according to manufacturers’ instructions, SDS and workplace requirements. |
|  |  | 2.4 | Clean, safety-check and store vehicle equipment to enable ready access and re-use and record identified maintenance requirements. |
|  |  | 2.5 | Maintain storage area free from obstacles and collect and dispose of waste according to workplace and compliance requirements. |
| 3 | Control and maintain stocks of equipment and chemicals. | 3.1 | Operate and maintain inventory system with maximum and minimum holding levels of equipment and chemicals according to workplace requirements. |
|  |  | 3.2 | Monitor equipment and chemical use patterns and reorder stock to maintain required levels. |
|  |  | 3.3 | Prepare accurate and legible chemical labels with essential safety information for containers holding decanted and diluted pesticides. |
|  |  | 3.4 | Maintain an accurate labelling system for all containers according to workplace and compliance requirements. |
|  |  | 3.5 | Monitor chemical expiration dates and use chemicals on an oldest-manufactured, first-out basis. |
|  |  | 3.6 | Dispose of obsolete chemicals and equipment according to manufacturers’ instructions and compliance requirements. |
|  |  | 3.7 | Systematically complete stock control records and chemical register or manifest according to workplace and compliance requirements. |
| 4 | Control storage area during provision of pest management services. | 4.1 | Secure chemicals to prevent accidental spillage according to manufacturers’ instructions and compliance requirements. |
|  |  | 4.2 | Estimate, store and transport quantities of chemicals required for immediate pest management work according to manufacturers’ instructions and compliance requirements. |
|  |  | 4.3 | Store PPE separately to chemicals and vehicle occupants. |
|  |  | 4.4 | Secure, use and maintain vehicle equipment and PPE during pest management service provision. |
|  |  | 4.5 | Monitor pest management vehicle during service provision to restrict access and operation to authorised personnel according to workplace requirements |
| 5 | Maintain pest management vehicle. | 5.1 | Safety-check vehicle and organise regular servicing to ensure roadworthiness according to manufacturers’ instructions, workplace and compliance requirements. |
|  |  | 5.2 | Clean vehicle to remove chemical contaminants and dispose of waste according to workplace and compliance requirements. |
|  |  | 5.3 | Maintain vehicle to meet workplace presentation requirements. |
| 6 | Respond to emergency situations. | 6.1 | Identify accidental spillage or other emergency situation and promptly implement emergency response procedures according to workplace and compliance requirements. |
|  |  | 6.2 | Complete required compliance and workplace documentation detailing emergency situation and response. |

# Foundation Skills

Candidates require:

reading skills to interpret key requirements of equipment manuals, safety warning signs and symbols, pesticide labels and SDS

numeracy skills to:

calculate maximum and minimum holding levels of equipment and chemicals

interpret numerical information from SDS, manufacturers’ instructions and pesticide labels

writing skills to legibly and accurately complete chemical labels, stock control records and chemical registers or manifests.

# Unit Mapping Information

Supersedes and equivalent to CPPPMT3018 Maintain equipment and pesticide storage area in pest management vehicles.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=6f3f9672-30e8-4835-b348-205dfcf13d9b