# FBPBEV3004 Operate and monitor a beer filtration process

Release: 1

FBPBEV3004 Operate and monitor a beer filtration process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a beer filtration process to make bright beer.

This unit applies to individuals who have responsibility for filtering beer in a brewery. It typically applies to the production worker responsible for applying basic operating principles to the operation and monitoring of a beer filtration process and associated equipment.

All work must be carried out to comply with workplace procedures, in accordance with state/territory health and safety and food safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Beverages (BEV)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare the filtration process for operation | 1.1 Confirm beer is available to meet production requirements 1.2 Confirm availability of services according to operating requirements 1.3 Wear appropriate personal protective clothing, and ensure correct fit 1.4 Conduct pre-start checks of equipment according to operator instructions, including checking that hygiene and sanitisation standards are met 1.5 Set the filtration process to meet production requirements |
| 2. Operate and monitor equipment | 2.1 Start up beer filtration equipment according to safe operating procedures 2.2 Monitor equipment and control points to minimise dissolved oxygen, and confirm performance is maintained within specification 2.3 Undertake blending of beer as part of filtration process, as required of product specification 2.4 Confirm system and sub-system outputs meet specifications 2.5 Undertake microbiology testing at control points for risk of bacterial infection or wild yeast contamination 2.6 Identify, rectify and report out-of-specification product, process and equipment performance 2.7 Record production and other workplace information according to workplace procedures |
| 3. Shut down the filtration system | 3.1 Shut down the system safely according to operating procedures 3.2 Clean and maintain equipment to meet the workplace cleaning schedule and procedural requirements 3.3 Collect, treat and dispose of or recycle waste generated by both the process and cleaning procedures 3.4 Record workplace information according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace and standard operating procedures relevant to work task Interpret production requirements |
| Writing | Complete processing records using electronic, digital and/or paper-based formats |
| Numeracy | Monitor and interpret production and process control indicators and data Use fractions, decimals, proportions and percentages to determine responses to results Use volumes (ml, L, hL) and weights (mg, g, kg) to describe product quantities |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPBEV3004 Operate and monitor a beer filtration process | FBPOPR2013 Operate a bright beer tank process | Unit merged with FBPOPR2015 Operate a beer filtration process Prerequisite removed Unit title and sector code updated to better match work task Code updated to better match complexity of work task Minor changes to Performance Criteria to clarify intent Foundation Skills refined Performance Evidence clarified Minor changes to Knowledge Evidence and Assessment Conditions | Not equivalent |
| FBPBEV3004 Operate and monitor a beer filtration process | FBPOPR2015 Operate a beer filtration process | Unit merged with FBPOPR2013 Operate a bright beer tank process Prerequisite removed Unit title and sector code updated to better match work task Code updated to better match complexity of work task Minor changes to Performance Criteria to clarify intent Foundation Skills refined Performance Evidence clarified Minor changes to Knowledge Evidence and Assessment Conditions | Not equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPBEV3004 Operate and monitor a beer filtration process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has set up, operated, adjusted and shut down beer filtration equipment to process at least one batch of beer to specifications, including:

applying safe work practices

applying food safety procedures to work practices

using fobbing equipment correctly to control oxygen and beer quality

undertaking required corrective action in the event of variations to specification and procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of the filtration process

relationship between the wort production process, the fermentation process and bright beer tanks

stages and changes that occur during filtration

purpose of filter aids, filtration beds and filters in the beer production process

significance and methods of monitoring control points within the filtration process

beer filtration equipment used, including components, purpose and operation

consequences of turbulence and the procedures to avoid it

effect of bacteria, oxygen and temperature on quality

fobbing and its effect on quality

addition of flavours in filtration

addition of hop products in filtration

common causes of variation, and corrective action required

contamination/cross-contamination and food safety risks related to the filtration process, and related controls

good manufacturing practices (GMP) relevant to work task

health and safety hazards and controls relevant to the beer filtration process

environmental issues and controls, including waste handling requirements and procedures

shutdowns and cleaning requirements associated with changeovers

recording requirements and procedures for the beer filtration process

cleaning and sanitisation procedures for beer filtration equipment

sampling and testing procedures.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a brewery or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

relevant hazard control equipment and storage areas for hazardous goods

filtration equipment

filter aids and stabilising agents

beer to be filtered

specifications:

workplace procedures, including safe work practices, food safety, quality and environmental requirements

specifications, control points and parameters for the beer filtration equipment and process

beer filtration equipment operating and service manuals

recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4