# Assessment Requirements for FBPBPG2003 Operate the carton packing process

Release: 1

Assessment Requirements for FBPBPG2003 Operate the carton packing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has safely and effectively prepared for, started up, operated and shut down all items of equipment in a carton packing process on a minimum of one occasion, including:

confirming equipment is ready for operation, including:

multi-lane bottle diverters on infeed conveyors are operational

vacuum is operational

any adjacent coding equipment is set up and operational

line controls (conveyors) are operational

line lube is operational and being correctly applied

all hot melt guns or jets are operational

equipment is set for correct carton, cask and bottle dimensions

check weigher is operational

monitoring and adjusting process equipment to achieve required quality outcomes, ensuring:

compressed air pressure is at correct level

vacuum pressure is at correct level

hot melt units are maintained with correct adhesive level

carton supply is maintained

carton inserts are maintained

vacuum cups are picking up properly

hot glue strokes are correct length and producing a firm seal

weigher is rejecting underweight (short supplied) cartons

taking corrective action in response to typical faults and inconsistencies

liaising with other work areas involved in the packaging process

completing workplace records as required

applying safe work practices and identifying health and safety in the workplace hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of carton packing equipment

key features of carton packing equipment and components

links to related processes

stages and changes which occur during the process

effect of process stages on end product

quality characteristics and uses of end product

effect of product and materials on process outcomes

emergency and troubleshooting procedures

process specification, procedures and operating parameters

equipment and instrumentation components, purpose and operation

basic operating principles of process control systems where relevant

services required

significance and methods of monitoring control points

common causes of variation and corrective action required

technological advances that include automation and use of robots

health and safety in the workplace hazards and controls

routine maintenance requirements

lock-out and tag-out procedures

procedures and responsibility for reporting problems

shutdown and cleaning requirements associated with changeovers and types of shutdown

recording requirements and procedures.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a workplace setting or an environment that accurately represents a real workplace

resources, equipment and materials:

carton packing equipment

carton packing resources and consumables

tools and equipment for adjusting and maintaining equipment

personal protective equipment

specifications:

work procedures and operator manuals or instructions.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4