# Assessment Requirements for FBPBPG2004 Operate the palletising process

Release: 1

Assessment Requirements for FBPBPG2004 Operate the palletising process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has safely and effectively set up, operated and shut down the all items of equipment in the palletising process on a minimum of one occasion, including:

confirming equipment is ready for operation, including:

cleaning and sanitation processes are completed

vacuum is operational

pallet pattern is set to specifications

slip sheets for export are available

plastic shrink wrapper is loaded and operational

pallet identification equipment is operational

sufficient and correct pallets are available

carton turning equipment is operational

pallet tacking hot glue is operational

infeed guides and gates are operational

monitoring and adjusting process equipment to achieve required quality outcomes, ensuring:

compressed air pressure is at correct level

vacuum pressure is correct

cartons are going onto stripping plate according to set pattern

the ongoing quality and appearance of shrink wrapping

the ongoing quality and appearance of pallet identification

the ongoing quality and appearance of pallets and slip sheets

stacks are square on pallets

the ongoing quality and appearance of stacked pallets

taking corrective action in response to typical faults and inconsistencies

liaising with other work areas involved the packaging process

completing workplace records as required

applying safe work practices and identifying health and safety in the workplace hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of palletising equipment

key features of palletising equipment and components

links to related processes

effect of process stages on end product

quality characteristics and uses of end product

effect of product and materials on process outcomes

emergency and troubleshooting procedures

process specification, procedures and operating parameters

equipment and instrumentation components, purpose and operation

basic operating principles of process control systems where relevant

services required

significance and methods of monitoring control points

common causes of variation and corrective action required

health and safety in the workplace hazards and controls

routine maintenance requirements

lock-out and tag-out procedures

procedures and responsibility for reporting problems

shutdown and cleaning requirements associated with changeovers and types of shutdown

recording requirements and procedures.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a workplace setting or an environment that accurately represents a real workplace

resources, equipment and materials:

palletising equipment

palletising resources and consumables

tools and equipment for adjusting and maintaining equipment

personal protective equipment

specifications:

work procedures and operating instructions or instructions.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4