# Assessment Requirements for FBPBPG2006 Operate the bottle capsuling process

Release: 1

Assessment Requirements for FBPBPG2006 Operate the bottle capsuling process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has safely and effectively set up, operated and shut down all items of equipment in the bottle capsuling process on at least one occasion, including:

confirming equipment is ready for operation, including:

cleaning and sanitation processes are completed

capsule magazines, wax disc hoppers and wax pots are operational

any adjacent coding equipment is set up and operational

line controls (conveyors) are operational

line lube is operational and being correctly applied

heat shrink settings are adjusted to suit capsule type

metal-spinner tension is correct for capsule type

equipment is set for correct bottle height

hot melt for wax disc application is operational

bottle feeds are filled with correct screws and stars and accurately aligned

monitoring and adjusting process equipment to achieve required quality outcomes, ensuring:

compressed air pressure is at correct level

supply and quality of bottles to be capsuled

fill heights of bottles to be capsuled

clarity of content in bottles being capsuled

ongoing quality and appearance (shrinkage and fit) of capsule application

ongoing quality of materials used in the process

glue application and supply to wax disc process

taking corrective action in response to typical faults and inconsistencies

liaising with other work areas involved in the packaging process

completing workplace records as required

applying safe work practices and identifying workplace health and safety hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of bottle capsuling equipment

key features of bottle capsuling equipment and components

links to related processes

effect of process stages on end product

quality characteristics and uses of end product

effect of product and materials on process outcomes

emergency and troubleshooting procedures

process specification, procedures and operating parameters

equipment and instrumentation components, purpose and operation

basic operating principles of process control systems where relevant

services required

significance and methods of monitoring control points

common causes of variation and corrective action required

technological advances that include automation, such as use of robots

health and safety in the workplace hazards and controls

routine maintenance requirements

lock-out and tag-out procedures

procedures and responsibility for reporting problems

shutdown and cleaning requirements associated with changeovers and types of shutdown

recording requirements and procedures.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a workplace setting or an environment that accurately represents a real workplace

resources, equipment and materials:

bottle capsuling equipment

bottle capsuling resources and consumables

tools and equipment for adjusting and maintaining equipment

personal protective equipment

specifications:

work procedures and operating instructions or instructions.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4