# Assessment Requirements for FBPBPG3001 Operate the bottle filling process

Release: 1

Assessment Requirements for FBPBPG3001 Operate the bottle filling process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has safely and effectively set up, operated and shut down an automated or semi-automated bottle filling system on a minimum of one occasion, including:

confirming equipment is ready for operation, including:

cleaning and sanitation processes are completed

filling tubes meet specification

doser is set up and is operational

bottle feeds are filled with correct screws and stars and accurately aligned

height of filler is adjusted to suit bottle height

lines and equipment are flushed with product to be bottled

monitoring and adjusting process equipment to achieve required quality outcomes, ensuring:

compressed air pressure is at correct level

vacuum pressure is at correct level

product temperature

fill height and volume

taking corrective action in response to typical faults and inconsistencies

liaising with other work areas involved the packaging process

completing workplace records as required

applying safe work practices and identifying health and safety in the workplace hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of bottle filling equipment

key features of bottle filling equipment and components

links to related processes

effect of process stages on end product

quality characteristics and uses of end product

effect of product and materials on process outcomes

emergency and troubleshooting procedures

process specification, procedures and operating parameters

equipment and instrumentation components, purpose and operation

basic operating principles of process control systems where relevant

significance and methods of monitoring control points

common causes of variation and corrective action required

technological advances that include automation and use of robots

health and safety in the workplace hazards and controls

routine maintenance requirements

lock-out and tag-out procedures

procedures and responsibility for reporting problems

shutdown and cleaning requirements associated with changeovers and types of shutdown

recording requirements and procedures.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a workplace setting or an environment that accurately represents a real workplace

resources, equipment and materials:

bottling equipment

resources, tools and consumables

personal protective clothing and equipment

products, consumables and materials for filling purposes

cleaning procedures, materials and equipment

documentation and recording requirements and procedures

specifications:

specifications and schedules

equipment operating instructions, service records and corresponding information

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4