# Assessment Requirements for FBPBPG3004 Operate the bottle sealing process

Release: 1

Assessment Requirements for FBPBPG3004 Operate the bottle sealing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has safely and effectively set up, operated and shut down all items of equipment on a bottle sealing process on a minimum of one occasion, including:

confirming equipment is ready for operation, ensuring:

cleaning and sanitation processes are completed

magazine, hoppers, chutes and guides are operational

any adjacent coding equipment is set up and operational

line controls (conveyors) are operational

line lube is operational and being correctly applied

torque testers are correctly calibrated

equipment is set for correct bottle height

vacuum pump is operational

vacuum block and corker jaws are correctly lubricated

bottle feeds are filled with correct screws and stars and accurately aligned

monitoring and adjusting process equipment to achieve required quality outcomes, ensuring:

compressed air pressure is at correct level

torque on Roll On Tamper Evident (ROTE) seals

cork depths as established by the workplace

effective seal on crown seals

correct vacuum is being applied to head space in bottle

wire muselet is consistently applied

appearance of sealing application to approved quality

quality materials are used in the sealing process

taking corrective action in response to typical faults and inconsistencies

liaising with other work areas involved the packaging process

completing workplace records as required

applying safe work practices and identifying health and safety in the workplace hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of bottle sealing equipment

key features of bottle sealing equipment and components

links to related processes

stages and changes which occur during the process

effect of process stages on end product

quality characteristics and uses of end product

effect of product and materials on process outcomes

emergency and troubleshooting procedures

process specification, procedures and operating parameters

equipment and instrumentation components, purpose and operation

basic operating principles of process control systems where relevant

services required

significance and methods of monitoring control points

common causes of variation and corrective action required

technological advances that include automation

health and safety in the workplace hazards and controls

routine maintenance requirements

lock-out and tag-out procedures

procedures and responsibility for reporting problems

shutdown and cleaning requirements associated with changeovers and types of shutdown

recording requirements and procedures

equipment operation and out-of-specification actions for:

compressed air pressure

torque on ROTE seals, Roll On Pilfer Proof, Lux

inappropriate cork depths

ineffective seal on crown seals

incorrect vacuum applied to head space in bottle

inconsistent application of wire muselet

poor appearance of sealing application

poor quality of dry goods used in the sealing process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a workplace setting or an environment that accurately represents a real bottling and packaging workplace

resources, equipment and materials:

bottle sealing equipment

bottle sealing resources and consumables

tools and equipment for adjusting and, maintaining equipment

personal protective equipment

specifications:

work procedures and operator’s manuals or instructions.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4