# Assessment Requirements for FBPCEL3019 Prepare and apply complex additions and finings

Release: 1

Assessment Requirements for FBPCEL3019 Prepare and apply complex additions and finings

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with Food, Beverage and Pharmaceutical Training Package Version 4.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has independently prepared, mixed and applied eight different additions and finings to juice or wine according to addition and fining work orders, including:

used one of the following additions:

yeast

malolactic bacteria

used at least four of the following additions:

ammonium bisulfite

caramel

ascorbic acid/erythorbic acid

diammonium phosphate (DAP)

grape concentrate

hydrogen peroxide

malic acid

malolactic bacteria

potassium sorbate

sulphur dioxide products

tannin

tartaric acid

yeast

yeast nutrients

used at least three of the following finings:

activated carbon

bentonite

copper sulphate

enzymes

gelatine

polyvinylpolypyrrolidone (PVPP)

egg white

isinglass

casien.

In doing so, the individual must have:

used at least three of the following media:

cold water

warm water

juice

wine

used at least two of the following vessels and devices:

scales and other measuring equipment

barrel

bucket

mixing tub

manual agitator

mechanical agitator

wine tank

used at least three of the following application methods:

pour or drip in over the top

pump in over the top

pump in through tank valve

via doorway

venturi via pump

used at least two of the following integration methods:

gas rummaging

in place agitation

manual mixing

pumping over

submergible mixing

valve to valve

completed accurate and timely records of additions and finings applied according to workplace requirements, including:

juice or wine identification

addition or fining identification

date, time and operator

quantity of material added by weight or volume.

After each addition and fining has been applied to a batch of juice or wine, the individual must have:

collected, labelled and transferred a sample to a laboratory for analysis or tasting, using at least two of the following methods:

over the top

sample tap or valve

in line sample during transfer.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

the physical properties, functions and benefits of wine additions, including:

ammonium bisulfite

ascorbic acid/erythorbic acid

caramel

hydrogen peroxide

grape concentrate

malic acid

malolactic bacteria

potassium sorbate

sulphur dioxide products

tannin

tartaric acid

yeast

yeast nutrients

the physical properties, functions and benefits of wine fining agents, including:

activated carbon

bentonite

casien

cream of tartar

copper sulphate

egg white

enzymes

gelatine

isinglass

polyvinylpolypyrrolidone (PVPP)

quality process and indicators related to the storage, mixing and application of additions and finings, including:

quarantine and testing of raw materials, including batch codes

sampling and testing of juice or wine prior to and after the application of additions and finings

contamination risks and controls associated with the preparation and application of additions and finings

requirements for vegan and allergen control, including label integrity requirements

weights and measures for additions and finings, including:

calibration of scales

units of measurement, including grams, kilograms, tonnes, millilitres, litres, hectolitres and kilolitres

mixing media used to prepare additions and finings used in wine making, including:

cold water

warm water

juice

wine

mixing vessels and devices used to prepare additions and finings used in wine making, including:

barrel

bucket

mixing tub

manual agitator

mechanical agitator

application methods for applying additions and finings to wine, including:

pour in over the top

pump in over the top

tank to tank transfer

valve to valve

venturi

equipment and methods for integrating additions and finings with wine or juice, including:

gas rummaging

in place mixing

manual mixing

submergible mixing

pumping over

valve to valve

record keeping requirements for additions and finings in wine making, including:

juice or wine identification

addition or fining identification

date, time and operator

quantity of material added by weight or volume

principles and methods for collection of wine samples for laboratory analysis, including:

over the top

sample tap

in line sample during transfer

cross-contamination

containers used

labelling

effects of time

workplace health and safety hazards and controls, including:

awareness of the limitations of controls

personal protective equipment

entering and working in confined spaces

hazardous substances, including additions and finings, cleaning products and gases

environmental issues and controls relevant to the storage, mixing and application of additions and finings, including:

rework and reuse of products

water use and recycling

waste processing

energy usage

materials and manual handling procedures related to the preparation, mixing and application of additions and finings.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a commercial winery

resources, equipment and materials:

the vessels, machinery and equipment stipulated in the performance evidence

the juice or wine stipulated in the performance evidence

the additions and finings stipulated in the performance evidence

specifications:

addition and fining work orders stipulated in the performance evidence

workplace procedures for preparation and application of the additions, finings, equipment and machinery stipulated in the performance evidence.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4