# Assessment Requirements for FBPCON2002 Operate a boiled confectionery process

Release: 1

Assessment Requirements for FBPCON2002 Operate a boiled confectionery process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0. |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit. There must be evidence that the individual has operated at least one high and low  boiled confectionery process, including:

checking the availability of materials

selecting, fitting and using personal protective equipment

conducting pre-start checks on machinery

starting, operating, monitoring and adjusting process equipment to achieve required quality outcomes

taking corrective action in response to typical faults and inconsistencies

completing workplace records

shutting down equipment.

Throughout the process, individuals must:

apply food safety procedures to work practices

follow work health and safety and equipment operating procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the high/low boil process relating to the range of products to be produced, including:

ingredients used and their purpose

syrup making (super saturated solutions)

relationship between temperature and moisture content

the effect of vacuum on processing

reactions including crystallisation

process stages

maturation and further processing requirements

basic operating principles of equipment, including main equipment components, status and purpose of guards, the effect of vacuum on the cooking process and further processing, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation

services required and action to take if services are not available

the flow of the high/low boil process and the effect of outputs on downstream processes

quality requirements of materials used in low and high boil manufacture and effect of variation on process performance and outputs

quality characteristics required of process outputs

methods used to further process high and low boils, including basic forming or extruding and stamping, cooling, pulling and aerating as required by product range

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the high/low boil process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination and food safety risks associated with the high and low boil process and related control measures

common causes of variation and corrective action required

work health and safety hazards and controls

requirements of different shutdowns as appropriate to the high and low boil process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock out and tag out procedures and responsibilities

product and process changeover procedures and responsibilities

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the high and low boil process, including waste and rework collection and handling procedures related to the process

basic operating principles of process control, where relevant, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures, where relevant.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

process and related equipment and services, including mixing equipment, cookers and coolers

ingredients used in the preparation of high/low boil product

sampling schedules and test procedures and equipment

cleaning materials, equipment and procedures

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

recipe/batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4