# FBPCON2004 Operate a chocolate depositing or moulding process

Release: 1

FBPCON2004 Operate a chocolate depositing or moulding process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0. |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a depositing process that deposits chocolate into moulds or directly onto conveyors.

This unit applies to individuals who work under general supervision as production workers and are responsible for the operation and monitoring of chocolate depositing or moulding equipment in a confectionery production environment.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory work health and safety, and food safety regulations, legislation and standards that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Confectionery (CON)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare the depositing equipment and process for operation | 1.1 Confirm materials are available according to workplace procedure 1.2 Identify safety requirements and wear appropriate personal protective equipment ensuring correct fit 1.3 Condition equipment and ingredients according to recipe 1.4 Identify and confirm cleaning and maintenance requirements 1.5 Fit and adjust machine components and related attachments according to equipment operating instructions 1.6 Enter processing or operating parameters according to recipe and equipment operating instructions 1.7 Carry out pre-start checks according to operating procedures |
| 2. Operate and monitor the depositing/moulding process | 2.1 Start and operate the process safely according to operating instructions and safety procedures 2.2 Deposit chocolate to achieve required coverage 2.3 Monitor equipment to identify variation in operating conditions 2.4 Identify variation in equipment operation and report maintenance requirements according to workplace procedures 2.5 Confirm that specifications are met at each stage with recipe 2.6 Identify, rectify or report out-of-specification product or process outcomes according to workplace procedures 2.7 Maintain the work area according to workplace procedures 2.8 Maintain workplace records according to workplace procedures |
| 3. Shut down the depositing/moulding process | 3.1 Identify the appropriate shutdown procedure according to operating instructions 3.2 Shut down the process safely according to shutdown procedures 3.3 Identify and report maintenance requirements according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interprets and comprehends information contained in chocolate depositing or moulding equipment operational instructions and workplace procedures |
| Writing | Enters operational data into production equipment accurately |
| Numeracy | Locates and applies basic calculations for numerical information found in recipes and production equipment |
| Navigate the world of work | Follows clear instructions within defined level of responsibility Recognises organisational expectations and follows explicit protocols and procedures, seeking clarification when required |
| Interact with others | Follows instructions and procedures regarding what and how to communicate when reporting on chocolate depositing or moulding production issues |
| Get the work done | Follows clearly defined instructions and sequencing, and monitors own progress for chocolate depositing or moulding production and seeks assistance when necessary Makes low-impact decisions around immediate clearly defined production tasks |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPCON2004 Operate a chocolate depositing or moulding process | FDFCON2004A Operate a chocolate depositing or moulding process | Updated to meet Standards for Training Packages Minor changes to Performance Criteria for clarity | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPCON2004 Operate a chocolate depositing or moulding process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0. |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit. There must be evidence that the individual has operated at least one chocolate depositing or moulding process, including:

checking the availability of materials

selecting, fitting and using appropriate personal protective equipment

conducting pre-start checks on machinery

starting, operating, monitoring and adjusting process equipment to achieve required quality outcomes

taking corrective action in response to typical faults and inconsistencies

completing workplace records

shutting down equipment.

Throughout the process, individuals must:

apply food safety procedures to work practices

follow work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the chocolate depositing process, including ingredients used, handling and conditioning requirements and process stages

basic operating principles of equipment, including main equipment components, status and purpose of guards, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation

services required and action to take if services are not available

the flow of the chocolate depositing process and the effect of outputs on downstream processes

quality characteristics and types of chocolate used for depositing

quality requirements of centres as appropriate to product and the effect of variation on process outputs

quality characteristics to be achieved by the deposited product

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the chocolate depositing process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the chocolate depositing process and related control measures

common causes of variation and corrective action required

work health and safety hazards and controls

requirements of different shutdowns as appropriate to the chocolate depositing process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock out and tag out procedures and responsibilities

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the chocolate depositing process, including waste and rework collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures for chocolate depositing or moulding equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

process and related equipment and services

ingredients used in the preparation of chocolate depositing or moulding product as required

sampling schedules and test procedures and equipment

cleaning materials, equipment and procedures

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

recipe/batch instructions for chocolate depositing or moulding

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4