# Assessment Requirements for FBPGRA4002 Supervise testing processes for wheat and flour

Release: 1

Assessment Requirements for FBPGRA4002 Supervise testing processes for wheat and flour

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that, on at least one occasion, the individual has:

identified and interpreted customer requirements

identified regulatory requirements for mill operation

identified mill critical control points

established sampling procedures and tests for grain

determined sampling procedures and rates for tests

established procedures for test equipment maintenance and accuracy

established procedures for storage and labelling of samples

integrated testing regime with HACCP system

maintained integrity of mill testing regime through use of control and reference tests and appropriate calibration and maintenance of test equipment

applied safe work practices and identified work health and safety hazards and controls.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

effect of reduction and break stages on flour characteristics, including:

protein level

protein quality

ash content

colour

starch proportion

dough rheology

test and test procedures appropriate to grain, including:

sample appearance and smell

sieving for sample purity

near infrared test (NIR) and other tests for moisture, protein content, colour and grain hardness

difference between ‘as is’ and ‘dry weight’ and moisture corrected moisture results and conversion of other parameters

gluten washing test for protein quality

Hagberg Falling Number test, including conversion of results to Perten Liquefaction Numbers

Kern chondrometer test and other specific weight tests

tests and test procedures related to flour, including:

NIR tests for flour moisture, protein content, colour, water absorption and ash content

colour testing using Kent-Jones and Martin flour colour grader

Pekar test

laboratory sieve tests

Brabender Extensograph and Farinograph test for dough stretching

viscosity tests

starch damage tests

Chittick test for carbon dioxide release

baking tests

reference tests used for calibration, including:

oven test for moisture

Zeleny test for protein quality

Single Kernel Characterisation System (SKCS) test

Stenvert test for hardness

electrophoresis test for variety

image analysis for particle size and colour

muffle furnace test for ash content

fibre tests

expected extraction rate for different stages of the milling process

Australian grain characteristics relevant to milling, including:

variety

wheat segregations, including soft, Australian Premium White (APW), Hard and Prime Hard (PH)

other Australian grain segregations

typical composition of bread, biscuit and cake-making flours, including moisture, sugar, starch, protein, fibre and mineral matter

flour characteristics giving rise to faults in bread, pasta or other flour based products, including:

incorrect flour strength

incorrect alpha-amylase activity

incorrect amount of damaged starch

bran contamination

incorrect improver dosage

heat damaged wheat

tainted flour

microbiology tests for grain and flour

tests used in manufacture of by-products.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment relevant to flour milling operations

sampling schedules and test procedures and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

equipment manuals including operating parameters

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4