# Assessment Requirements for FBPHVB3007 Manufacture crisp breads

Release: 1

Assessment Requirements for FBPHVB3007 Manufacture crisp breads

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has manufactured at least one batch of crisp breads to meet product specifications, including:

applying safe work practices

applying food safety procedures to work practices

confirming supply of necessary materials and services, including confirming slurry quality, quantity, temperature and degree of aeration

confirming settings, including:

ovens and cutter settings for product type

web speed settings

depositing rate settings

flour (dusting) process settings

docking roller adjusted for product type

monitoring and adjusting manufacturing equipment to achieve production specification, including:

slurry volume, weight and density

size (width, length and thickness)

texture, colour and weight

moisture level

clean, even break or cut

taking corrective action in response to typical faults and inconsistencies.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of each stage of the crisp breads manufacturing process

basic operating principles of equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

typical equipment faults and related causes, including signs and symptoms of faulty equipment, loss of power, and early warning signs of potential problems

the flow of the crisp breads manufacturing process, and the effect of outputs on downstream processes

basic operating principles of process control, where relevant, including the relationship between control panels and systems and the crisp bread manufacturing equipment

specifications and quality characteristics to be achieved by the crisp breads manufacturing process, including:

quality requirements of materials, and effect of variation on process performance

effect of slurry temperature on its ability to hold air

effect of docking roller on baking process

changes that occur in product during processing

good manufacturing practices (GMP) relevant to work task

contamination/cross-contamination, cross-contact allergen risks and food safety risks associated with manufacturing crisp breads, and related control measures

methods used to monitor the crisp breads manufacturing process, including:

inspecting, measuring and testing as required by the crisp bread manufacturing process

inspection or test points (control points) in the process

related procedures and recording requirements

common causes of variation, and corrective action required

health and safety hazards and controls

procedures and responsibility for reporting production and performance information

environmental and food safety issues and controls relevant to the crisp breads manufacturing process, including:

waste and rework collection and handling procedures

contamination and food safety risks

cleaning and sanitisation procedures

emerging technologies related to high volume production baking

procedures to track traceability of product.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a high volume production bakery or an environment that accurately represents workplace conditions

resources, equipment and materials:

dough mix and slurry to meet production requirements

personal protective equipment

depositing, baking, cutting and stacking equipment

cleaning procedures, materials and equipment

specifications:

production schedule and recipe/batch specifications

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4