# Assessment Requirements for FBPHVB3019 Operate a bread dough make up process

Release: 1

Assessment Requirements for FBPHVB3019 Operate a bread dough make up process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated a dough make up process to produce at least one batch of product to meet specifications, including:

applying safe work practices

applying food safety procedures to work practices

conducting inspections to confirm process conforms to specifications, including:

dough height in hopper

dough weight

oil flow to divider

size and shape of dough piece

skin formation during intermediate proof

presence of ‘doubles’

process variables such as time, temperature and humidity

taking corrective action in response to results that fail to meet specifications.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

the purpose and basic principles of each stage of the dough make up process

basic operating principles of equipment, including main equipment components, status and purpose of guards, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation

good manufacturing practices (GMP) relevant to work task

flow of the dough make up process, and the effect of outputs on downstream processes

effect of process variables, including time, temperature and humidity, on process stages

procedures for carrying out pre-start checks, including:

divider speed, weight setting and oil supply

air flow from fans in rounder

intermediate proving process airflow

moulding process settings, including sheeting, sealing board height and width, four piecing and roller adjustments

panning process settings, including side guides, sealing board, knives, folders and side belts

effect of process stages, including moulding, on dough structure

quality characteristics to be achieved by the dough make up process

quality requirements of materials, and effects of variation on the dough make up process

operating requirements and parameters, and corrective action required if operation falls outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods of monitoring dough make up process, such as inspecting, measuring and testing as required

inspection or test points (control points) in dough make up process, and the related procedures and requirements

contamination/cross-contamination, cross-contact allergen risks and food safety risks associated with dough make up, and related control measures

health and safety hazards and controls relevant to the work process, including the limitations of personal protective equipment

requirements of different shutdowns and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock-out or tag-out procedures and responsibilities

cleaning and sanitisation procedures for workspace and equipment

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the dough make up process, including waste collection and handling procedures related to the process

emerging technologies related to high volume production baking

procedures to track traceability of product.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a high volume production bakery or an environment that accurately represents workplace conditions

resources, equipment and materials:

dough to be processed

personal protective clothing and equipment

dough make up and transfer equipment and related services

specifications:

production schedule and batch instructions

work procedures, including advice on safe work practices, food safety, quality, cleaning and environmental requirements

information on equipment capacity and operating parameters

sampling schedules and test procedures and equipment.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4