# Assessment Requirements for FBPOIL3010 Operate and monitor a malaxer

Release: 1

Assessment Requirements for FBPOIL3010 Operate and monitor a malaxer

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated and monitored a malaxer to release oil from at least one batch of crushed olive paste, to meet specifications, including:

applying food safety practices

following safe work procedures

taking corrective action in response to typical faults and inconsistencies in the malaxing process.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of malaxing olive paste

Australian Standard for Olive Oils and Olive-Pomace Oils (AS 5264-2011)

Australian Olive Industry Code of Practice

basic operating principles of mixing/blending equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

awareness of calibration schedules for scales and related weighing/measuring equipment

services required, and action to take if services are not available

ingredient-handling requirements and shelf-life or coding

quality characteristics required of olive paste

the characteristics of pastes made from different types of olives

additives and enzymes that can be added to the agitation process to ensure enough oil is released, including: pectinases, cellulases and hemicellulases; talcum or micro talcum powder (hydrated magnesium silicate); water – these technological coadjuvants should not be detectable in the final product

the effect of temperature on the olive paste, and the need for paste to be kept at 25–30C

the effect of malaxation time on the efficiency of oil separation and oil quality

calculation of malaxation time

inspection or test points (control points) in the process and the related procedures and recording requirements, including times/temperatures and agitator speeds

contamination/cross-contamination and food safety risks associated with the process, and related control measures, including cross-contamination risks and associated cleaning requirements

operating requirements and parameters, and corrective action required when operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment, and early warning signs of potential problems

common causes of variation, and corrective action required

health and safety hazards and controls relevant to the mixing and blending process

requirements of different shutdowns as appropriate to the blending or mixing process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities relevant to the mixing and blending process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the malaxing process, including waste or rework collection and handling procedures related to the process

basic operating principles of process control, where relevant, including the relationship between control panels and systems and the physical equipment

cleaning and sanitisation procedures for mixing or blending equipment

recording procedures to ensure traceability of product.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

an olive oil processing workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

malaxer process equipment and related services

personal protective clothing and equipment

crushed olives

cleaning materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

batch/recipe instructions

information on equipment capacity and operating parameters

recording requirements.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4