# Assessment Requirements for FBPOIL3012 Operate and monitor storage of virgin olive oil

Release: 1

Assessment Requirements for FBPOIL3012 Operate and monitor storage of virgin olive oil

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated and monitored the storage of at least one batch of virgin olive oil, including:

applying safe work procedures

following food safety procedures

monitoring and adjusting the storage environment and equipment to achieve required outcomes

taking corrective action in response to typical changes in the storage environment.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

Australian Standard for Olive Oils and Olive-Pomace Oils (AS 5264-2011)

Australian Olive Industry Code of Practice

the flow of the holding and storage process and the effect of product output on downstream processes

contamination/cross-contamination and food safety risks associated with the holding and storage process, and related control measures

time required for oil to settle

settling of olive oil, including the processes of purging, racking-off and filtration

when filtration is required

ideal storage conditions for virgin olive oil, including temperature, container properties, length of time product is stored

organoleptic properties of different types of olive oil

factors that accelerate the breakdown of virgin olive oil, including:

exposure to air (oxidation)

heat

light (and using transparent containers)

contact with sediments and water (hydrolysis)

delayed processing of harvested fruit (causing fermentation)

processing damaged (high FFA) fruit (diseased, mummified, split, over-ripe, frosted), which will result in defective oils

variety and style (oxidative stability)

time

types of containers used to store bulk olive oil, and their characteristics

techniques used to monitor the holding and storage process, including inspecting, measuring and testing by the process

characteristics of different types of olive oil (extra virgin, virgin, refined, light, pomace)

sampling methods

typical tests undertaken to determine the quality of olive oil, including tests to detect alteration and refining processes, including soft deodorisation

parameters that identify olive oil as fresh, well processed and stored appropriately, including:

free fatty acids (FFAs) in the vicinity of ≤0.4

peroxide value (PV) in the vicinity of ≤12

K232 values under 2.00 and K268 values under 0.18

pyropheophytin a (PPP) <1 % of PPPs (which will increase 6–8 % per year, under ideal storage conditions)

around 90% of 1,2-diacylglycerols (DAG) (which will decrease 20–25 % per year, under ideal storage conditions)

common olive oil defects and how each is caused:

fusty

musty

winey (vinegary)

muddy (sediment)

rancid

methods for maintaining oil temperature during storage or distribution

health and safety hazards and controls, including the limitations of protective clothing and equipment relevant to the work process

environmental issues and controls relevant to the process, including disposal of waste water and defect products

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitisation procedures required for the storage containers, storage area and equipment

recording requirements to ensure traceability and biosecurity of product.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a food storage workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing

holding and storage equipment and related services

olive oil for storage/containers

sampling and testing equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

product specifications, control points and processing parameters

information on equipment capacity and operating parameters

sampling schedules and test procedures production

recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4