# FBPOIL3012 Operate and monitor storage of virgin olive oil

Release: 1

FBPOIL3012 Operate and monitor storage of virgin olive oil

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, monitor and maintain the storage of virgin olive oil to maintain product integrity.

This unit applies to individuals who work in an olive oil processing environment. It typically applies to the production worker responsible for operating and monitoring of a holding and storage process and associated equipment.

All work must be carried out to comply with workplace procedures, in accordance with state/territory health and safety and food safety regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Edible Oils (OIL)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work in storage area | 1.1 Inspect olive oil in receival tanks to ensure sediment has settled and water has been purged 1.2 Confirm storage containers or tanks are clean and of a suitable grade to store oil safely and to maintain quality, according to industry standards 1.3 Wear appropriate personal protective clothing and ensure correct fit 1.4 Rack-off purged oil into clean storage tanks, filtering where required 1.5 Seal storage tanks to avoid oxidation 1.6 Ensure the storage space is clean, according to workplace standards |
| 2. Operate and monitor storage | 2.1 Move containers of virgin olive oil to storage area using manual handling aids 2.2 Check temperature of olive oil to ensure it is less than 20C 2.3 Monitor the storage environment and equipment to ensure conditions remain ideal for product 2.4 Identify, adjust and report out-of-specification equipment, product or process outcomes according to workplace requirements 2.5 Follow and apply workplace housekeeping procedures and environmental requirements 2.6 Maintain workplace records according to workplace requirements |
| 3. Monitor stored product | 3.1 Monitor the virgin olive oil to confirm that storage conditions maintain the quality of oil 3.2 Take samples and test product to ensure specifications are met 3.3 Maintain batch records of product, according to workplace requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace and standard operating procedures relevant to work task Interpret storage requirements |
| Writing | Record storage and product information using digital and/or paper-based format |
| Numeracy | Monitor and interpret storage control indicators including time, temperature, volume (L, hL) Interpret test results, including free fatty acid (FFA) (% m/m), oil oxidation, peroxide value (PV) (meq O2/kg oil) |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOIL3012 Operate and monitor storage of virgin olive oil | Not applicable | The unit has been created to address a skill or task required by industry that is not covered by an existing unit | Newly created |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPOIL3012 Operate and monitor storage of virgin olive oil

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated and monitored the storage of at least one batch of virgin olive oil, including:

applying safe work procedures

following food safety procedures

monitoring and adjusting the storage environment and equipment to achieve required outcomes

taking corrective action in response to typical changes in the storage environment.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

Australian Standard for Olive Oils and Olive-Pomace Oils (AS 5264-2011)

Australian Olive Industry Code of Practice

the flow of the holding and storage process and the effect of product output on downstream processes

contamination/cross-contamination and food safety risks associated with the holding and storage process, and related control measures

time required for oil to settle

settling of olive oil, including the processes of purging, racking-off and filtration

when filtration is required

ideal storage conditions for virgin olive oil, including temperature, container properties, length of time product is stored

organoleptic properties of different types of olive oil

factors that accelerate the breakdown of virgin olive oil, including:

exposure to air (oxidation)

heat

light (and using transparent containers)

contact with sediments and water (hydrolysis)

delayed processing of harvested fruit (causing fermentation)

processing damaged (high FFA) fruit (diseased, mummified, split, over-ripe, frosted), which will result in defective oils

variety and style (oxidative stability)

time

types of containers used to store bulk olive oil, and their characteristics

techniques used to monitor the holding and storage process, including inspecting, measuring and testing by the process

characteristics of different types of olive oil (extra virgin, virgin, refined, light, pomace)

sampling methods

typical tests undertaken to determine the quality of olive oil, including tests to detect alteration and refining processes, including soft deodorisation

parameters that identify olive oil as fresh, well processed and stored appropriately, including:

free fatty acids (FFAs) in the vicinity of ≤0.4

peroxide value (PV) in the vicinity of ≤12

K232 values under 2.00 and K268 values under 0.18

pyropheophytin a (PPP) <1 % of PPPs (which will increase 6–8 % per year, under ideal storage conditions)

around 90% of 1,2-diacylglycerols (DAG) (which will decrease 20–25 % per year, under ideal storage conditions)

common olive oil defects and how each is caused:

fusty

musty

winey (vinegary)

muddy (sediment)

rancid

methods for maintaining oil temperature during storage or distribution

health and safety hazards and controls, including the limitations of protective clothing and equipment relevant to the work process

environmental issues and controls relevant to the process, including disposal of waste water and defect products

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitisation procedures required for the storage containers, storage area and equipment

recording requirements to ensure traceability and biosecurity of product.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a food storage workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing

holding and storage equipment and related services

olive oil for storage/containers

sampling and testing equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

product specifications, control points and processing parameters

information on equipment capacity and operating parameters

sampling schedules and test procedures production

recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4