# Assessment Requirements for FBPOPR2029 Operate a baking process

Release: 1

Assessment Requirements for FBPOPR2029 Operate a baking process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a baking process on at least one occasion, including:

accessing workplace information to identify baking process requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

confirming condition and quality of ingredients

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational, and rework bins are positioned

confirming settings and parameters, including:

correct oven settings for baking time

oven zone temperatures

conveyor speed, loaders and dischargers

circulation and exhaust fans

water seals

burner controls and steam

heights, guides and air blowers

pre-blast conveyor

starting, operating, monitoring and adjusting process equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including:

baking output, including quantities and product appearance (colour and sheen, uniformity of shape, weight and size, and moisture)

confirming product is cleanly removed from the pan (depanning stage) and/or belt/band

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on baking equipment

following isolation and lock-out/tag-out procedures to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

sorting, collecting, treating, recycling or disposing of waste, including clearing dust extractors

completing workplace records

maintaining work area to meet housekeeping standards

setting up, starting up and monitoring the proofer, including time and temperature, humidity and position of the loading arm

monitoring the proving process according to workplace procedures, including dough condition/surface and dough size and height

setting up and operating finishing equipment according to workplace procedures, including setting topping application equipment, water sprays and splitters (monitoring typically involves checking appearance to confirm even coverage and/or finish to product specification)

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of each stage of the baking process, including the stages of the baking process and changes that occur in the product, including starch gelatinisation, the effect of variables (e.g. time, temperature and humidity), and oven zones as appropriate to oven type

basic operating principles of equipment, including:

the difference between convection and impingement ovens

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

services required and action to take if services are not available

the flow of the baking process and the effect of outputs on customer satisfaction and downstream processes, including packaging

quality requirements of materials and the effect of variation on baking process performance

quality characteristics to be achieved by the baked product

process specifications, procedures and operating parameters for different baked pastry products

operating requirements, parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the baking process, including inspecting, measuring and testing, by the process

inspection or test points (control points) in the baking process and the related procedures and recording requirements, including the ability to conduct dew point and/or wet bulb tests

contamination/food safety risks associated with the baking process and related control measures

common causes of variation and corrective action required

work health and safety hazards and controls, including risks of spontaneous combustion and limitations of protective clothing and equipment used

requirements of different shutdowns as appropriate to the baking process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities relevant to operating a baking process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the baking process, including waste/rework collection and handling procedures related to the process

purpose and operating procedures for related processes, including in-line proving and finishing equipment where relevant

basic operating principles of process control where relevant, including the relationship between control panels and systems and the physical equipment

routine maintenance procedures for baking equipment

cleaning and sanitation procedures for baking equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

baking and depanning equipment and services, and related equipment

product to be baked

sampling schedules and test procedures and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

specifications, control points and processing parameters

production schedule and recipe/batch instructions

documentation and recording requirements and procedures

cleaning procedures, materials and equipment.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4