# Assessment Requirements for FBPOPR2034 Operate an evaporation process

Release: 1

Assessment Requirements for FBPOPR2034 Operate an evaporation process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated an evaporation process on at least one occasion, including:

accessing workplace information to identify processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

preparing materials for an evaporation process

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs as required

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting evaporation equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, including:

temperatures

vacuum

motor amperage

condensate flow

steam flow and pressure

throughput

time/speed

evaporated product characteristics

monitoring supply and flow of materials to and from the evaporation process

taking corrective action in response to out-of-specification results

reporting and/or recording corrective action as required

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures as required to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

preparing equipment for cleaning

responding to and/or reporting equipment failure within level of responsibility

completing workplace records as required

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of evaporation, including:

an understanding of the principles of steam

the relationship between boiling point and pressure as applied to evaporation

the changes that occur to product as it moves through the evaporation process

basic operating principles of evaporation equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

quality characteristics to be achieved by the evaporation process

the flow of the evaporation process and the effect of product output on downstream processes

quality requirements of materials and the effect of variation on process performance

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

techniques used to monitor the evaporation process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the process and related control measures relevant to the evaporation process

common causes of variation and corrective action required

work health and safety hazards and controls relevant to the evaporation process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the evaporation process, including waste/rework collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures for evaporation equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

evaporation process and related equipment and services

materials to be evaporated

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule, batch/recipe instructions

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4