# FBPOPR2035 Operate an enrobing process

Release: 1

FBPOPR2035 Operate an enrobing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down an enrobing process.

This unit applies to individuals who work under general supervision and exercise limited autonomy in a food processing environment. It typically applies to the production worker responsible for applying basic operating principles to the operation and monitoring of an enrobing process and associated equipment.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory work health and safety, and food safety regulations, legislation and standards that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Operational (OPR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work in processing area | 1.1 Confirm tempered chocolate is available for enrobing, and condition centres/bars as required to meet operating requirements 1.2 Identify and confirm cleaning and maintenance requirements and status according to work health and safety and food safety requirements 1.3 Wear appropriate personal protective clothing and ensure correct fit |
| 2. Prepare the enrobing equipment and process for operation | 2.1 Fit and adjust machine components and related attachments to meet operating requirements 2.2 Enter processing/operating parameters as required to meet safety and production requirements 2.3 Check and adjust equipment performance according to operating requirements 2.4 Carry out pre-start checks according to work health and safety and operating requirements |
| 3. Operate and monitor the enrobing process | 3.1 Start, operate and monitor the enrobing process for variation in operating conditions 3.2 Monitor the process to confirm that specifications are met 3.3 Identify, rectify and report out-of-specification equipment, product and process outcomes according to workplace requirements 3.4 Follow and apply workplace housekeeping and environmental procedures |
| 4. Shut down the enrobing process | 4.1 Identify the appropriate shutdown procedure 4.2 Shut down the process according to work health and safety and operating requirements 4.3 Identify and report maintenance requirements according to workplace procedures 4.4 Maintain workplace records according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace and standard operating procedures relevant to job role Interpret production requirements |
| Writing | Record operating, routine maintenance and shutdown information on paper-based and electronic media |
| Numeracy | Monitor and interpret production and process control indicators and data Monitor supply and flow of materials to and from the process |
| Navigate the world of work | Apply workplace procedures to own role and responsibilities Identify skills, knowledge and experience required for performing own role in the workplace Ask questions to clarify understanding or seek further information |
| Interact with others | Communicate and report operational and safety information to relevant personnel Use modes of communication suitable to purpose to confirm understanding when conducting operational start-up and shutdown procedures |
| Get the work done | Access and interpret process control panels and system Maintain a clean and hazard-free work area Identify out-of-specification standards and maintain quality specifications Confirm equipment status and condition Interpret process and equipment information to monitor process and respond to abnormal performance and variations |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOPR2035 Operate an enrobing process | FDFOP2035A Operate an enrobing process | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPOPR2035 Operate an enrobing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated an enrobing process on at least one occasion, including:

accessing workplace information to identify enrobing process requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of tempered chocolate and the relevant centres/biscuits are available at the correct temperature

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs as required

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting enrobing process equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, including:

supply of chocolate to the enrobing reservoir

temperatures of chocolate and pipelines

chocolate temper

chocolate supply pump speed

curtain flow

blower and vibrator settings

temperature profile of cooling tunnel

chocolate coverage

backing off and tailing

product weight

appearance

cooling temperatures

monitoring supply and flow of materials to and from the enrobing process and equipment operation to confirm process remains within specification

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures as required to take enrobing process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

completing workplace records as required

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the enrobing process, including the effect of chocolate viscosity on enrobing, and types of chocolate suitable/used for coverage and backing off

basic operating principles of equipment used for the enrobing process, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

services required for the enrobing process and action to take if services are not available

the flow of the enrobing process and the effect of outputs on downstream processes

conditioning requirements for centres/bars/biscuits and chocolate, including understanding of temper test procedures

quality characteristics of centres/bars/biscuits and chocolate for use in the enrobing process and the effect of variation on process performance and outputs

quality characteristics to be achieved by the enrobing process

operating requirements and parameters, and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the enrobing process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the enrobing process and related control measures

common causes of variation for the operation of the enrobing process and corrective action required

work health and safety hazards and controls relevant to the operation of the enrobing process

requirements of different shutdowns as appropriate to the enrobing process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of the enrobing process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the enrobing process, including waste/rework collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitising procedures required for the enrobing process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

process and related equipment and services, including enrober and cooling equipment

materials to be processed, including tempered chocolate and conditioned centres to be enrobed

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

recipe/batch instructions

specifications, control points and processing parameters

sampling schedules and test procedures and equipment.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4