# Assessment Requirements for FBPOPR2039 Operate a frying process

Release: 1

Assessment Requirements for FBPOPR2039 Operate a frying process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a frying process on at least one occasion, including:

accessing workplace information to identify frying requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary oil type to meet product frying requirements, product to be processed and services

preparing materials as required prior to frying

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs as required

confirming that equipment is clean and valves are correctly positioned for oil transfer and processing stages

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting the frying process and equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, including:

product throughput/belt speeds

oil levels/temperatures

operation of oil filtration process

visual appearance of product, including breaking open to inspect

monitoring supply and flow of materials to and from the process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures as required to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

completing workplace records as required

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of frying, including optimal frying conditions and the role of oil filtration

basic operating principles of equipment used in the operation of a frying process, including:

main equipment components

the configuration of valves, pumps and pipework

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

the flow of the frying process and the effect of product output on downstream processes

quality characteristics to be achieved by the frying process

effect of product and oil quality characteristics on the frying process, final product and shelf-life

operating requirements and parameters, and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the frying process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the frying process and related control measures

common causes of variation related to the frying process and corrective action required

work health and safety hazards and controls, including the fire suppression system and procedures to follow if temperatures exceed the specified range

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of the frying process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the frying process, including procedures for recycling oil and for disposing of spent oil, and the consequences of incorrect disposal

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures required for the frying process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

in-line frying equipment and related equipment and services

frying oil suited to product type and product to be fried

sampling schedules and test procedures and equipment as required

cleaning procedures, materials and equipment as required

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule/batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4