# Assessment Requirements for FBPOPR2040 Operate a heat treatment process

Release: 1

Assessment Requirements for FBPOPR2040 Operate a heat treatment process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a heat treatment process on at least one occasion, including:

accessing workplace information to identify heat treatment requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

preparing materials

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting the heat treatment process and equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including:

time and temperature

pressure

flow rates

flow diversion

monitoring supply and flow of materials to and from the process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

cleaning and sanitising equipment

completing workplace records

maintaining work area to meet housekeeping standards

controlling related holding and cooling stages according to workplace procedures

using process control systems according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of heat treatment, including the effect of heat treatment on product and on microbiological characteristics

the relationship between time, temperature and pressure in the heat treatment process, including associated holding and cooling profiles according to product requirements

basic operating principles of equipment used in the heat treatment process, including:

main equipment components

types and characteristics of heating mediums used

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

the flow of the heat treatment process and the effect of product output on downstream processes

quality characteristics required of the heat-treated product

effect of raw material characteristics on the heat treatment process, including variation in viscosity/texture, microbial load and acidity

heat treatment requirements for food to be treated, including low and/or high acid foods

operating requirements and parameters for the heat treatment process, and corrective action required where operation is outside specified operating parameters

typical equipment faults for the heat treatment process and related causes, including:

signs and symptoms of faulty equipment

early warning signs of potential problems

methods used to monitor the heat treatment process, including inspecting, measuring and testing by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the process and related control measures

common causes of variation in the operation of the heat treatment process and corrective action required

work health and safety hazards and controls, including the limitations of protective clothing and equipment relevant to the work process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of the heat treatment process

procedures and responsibility for reporting production and performance information

cleaning and sanitation procedures required for heat treatment process

environmental issues and controls relevant to the heat treatment process, including waste/rework collection and handling procedures related to the process

basic operating principles of process control, where relevant, including the relationship between control panels and systems and the physical equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

heat treatment process and related equipment and services

materials to be heat treated

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule/batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4