# FBPOPR2043 Operate an homogenising process

Release: 1

FBPOPR2043 Operate an homogenising process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a homogenising process.

This unit applies to individuals who work under general supervision and exercise limited autonomy in a food processing environment. It typically applies to the production worker responsible for applying basic operating principles to the operation and monitoring of a homogenising process and associated equipment.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory work health and safety, and food safety regulations, legislation and standards that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Operational (OPR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work in processing area | 1.1 Confirm materials are available to meet operating requirements 1.2 Identify and confirm cleaning and maintenance requirements and status according to work health and safety and food safety requirements 1.3 Wear appropriate personal protective clothing and ensure correct fit |
| 2. Prepare homogenising process for operation | 2.1 Fit machine components and related attachments and adjust to meet operating requirements 2.2 Enter processing and operating parameters as required to meet safety and production requirements 2.3 Check equipment performance and adjust according to production requirements 2.4 Carry out pre-start checks according to work health and safety and operating procedures |
| 3. Operate and monitor the homogenising process | 3.1 Start, operate and monitor the process for variation in operating conditions 3.2 Monitor the process to confirm that specifications are met 3.3 Identify, rectify and report out-of-specification equipment, product or process outcomes according to workplace requirements 3.4 Follow and apply workplace housekeeping and environmental procedures |
| 4. Shut down homogenising equipment | 4.1 Identify the appropriate shutdown procedure 4.2 Shut down the process according to work health and safety and operating procedures 4.3 Identify and report maintenance requirements according to workplace procedures 4.4 Maintain workplace records according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace and standard operating procedures relevant to job role Interpret production requirements |
| Writing | Record operating, routine maintenance and shutdown information on paper-based and electronic media |
| Numeracy | Monitor and interpret production and process control indicators and data Monitor supply and flow of materials to and from the process |
| Navigate the world of work | Apply workplace procedures to own role and responsibilities Identify skills, knowledge and experience required for performing own role in the workplace Ask questions to clarify understanding or seek further information |
| Interact with others | Communicate and report operational and safety information to relevant personnel Use modes of communication suitable to purpose to confirm understanding when conducting operational start-up and shutdown procedures |
| Get the work done | Access and interpret process control panels and system Maintain a clean and hazard-free work area Identify out-of-specification standards and maintain quality specifications Confirm equipment status and condition Interpret process and equipment information to monitor process and respond to abnormal performance and variations |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOPR2043 Operate an homogenising process | FDFOP2043A Operate an homogenising process | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPOPR2043 Operate an homogenising process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated an homogenising process on at least one occasion, including:

accessing workplace information to identify processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs as required

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting process equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm the homogenising process remains within specification, including:

temperature

pressure

throughput

monitoring supply and flow of materials to and from the process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures as required to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

completing workplace records as required

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of homogenising

basic operating principles of equipment used in the operation of a homogenising process, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

effect of raw materials on homogenisation, including variables, including solids (brix), acidity, temperature, consistency and colour on process outcomes

quality requirements to be achieved by the homogenisation process

the flow of the homogenising process and the effect of product output on downstream processes

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults related to the homogenising process and related causes, including:

signs and symptoms of faulty equipment

early warning signs of potential problems

techniques used to monitor the homogenising process, including inspecting, measuring and testing as required by the process

inspection or test points (control points) in the process, and the related procedures and recording requirements

common causes of variation in the operation of the homogenising process and corrective action required

work health and safety hazards and controls relevant to the operation of the homogenising process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of the homogenising process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the homogenising process, including waste/rework collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures required for the operation of the homogenising process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

homogenisation process and related equipment and services

materials to be homogenised

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule/batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4