# FBPOPR2050 Operate a separation process

Release: 1

FBPOPR2050 Operate a separation process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a separation process.

This unit applies to individuals who work under general supervision and exercise limited autonomy in a food processing environment. It typically applies to the production worker responsible for applying basic operating principles to the operation and monitoring of a separation process and associated equipment.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory work health and safety, and food safety regulations, legislation and standards that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Operational (OPR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work in processing area | 1.1 Confirm materials are available to meet operating requirements 1.2 Identify and confirm cleaning and maintenance requirements and status according to work health and safety and food safety requirements 1.3 Wear appropriate personal protective clothing and ensure correct fit |
| 2. Prepare the separation process for operation | 2.1 Fit machine components and related attachments and adjust to meet operating requirements 2.2 Enter processing and operating parameters as required to meet safety and production requirements 2.3 Check equipment performance and adjust as required according to operating procedures 2.4 Carry out pre-start checks according to work health and safety and operating requirements |
| 3. Operate and monitor the separation process | 3.1 Start, operate and monitor the process for variation in operating conditions 3.2 Monitor the process to confirm that specifications are met 3.3 Identify, rectify and report out-of-specification equipment, product or process outcomes according to workplace requirements 3.4 Follow and apply workplace housekeeping and environmental procedures |
| 4. Shut down the separation process | 4.1 Identify the appropriate shutdown procedure 4.2 Shut down the process according to work health and safety and operating requirements 4.3 Identify and report maintenance requirements according to workplace procedures 4.4 Maintain workplace records according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret workplace and standard operating procedures relevant to job role Interpret production requirements |
| Writing | Record operating, routine maintenance and shutdown information on paper-based and electronic media |
| Numeracy | Monitor and interpret production and process control indicators and data Monitor supply and flow of materials to and from the process |
| Navigate the world of work | Apply workplace procedures to own role and responsibilities Identify skills, knowledge and experience required for performing own role in the workplace Ask questions to clarify understanding or seek further information |
| Interact with others | Communicate and report operational and safety information to relevant personnel Use modes of communication suitable to purpose to confirm understanding when conducting operational start-up and shutdown procedures |
| Get the work done | Access and interpret process control panels and system Maintain a clean and hazard-free work area Identify out-of-specification standards and maintain quality specifications Confirm equipment status and condition Interpret process and equipment information to monitor process and respond to abnormal performance and variations |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOPR2050 Operate a separation process | FDFOP2050A Operate a separation process | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPOPR2050 Operate a separation process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a separation process on at least one occasion, including:

accessing workplace information to identify separation processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

confirming that the correct bowl is fitted

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs as required

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting separation process equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, including:

separation speed

solids in in-feed and out-feed streams (this is typically done by in-line refractometers, Baume tests and spin tests)

flow rates

time/temperatures

pressures

air/water/oil levels

condition of seals and valves

discharge/desludging

monitoring supply and flow of materials to and from the separation process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

shutting down separation process equipment in response to emergency situation

following isolation and lock-out/tag-out procedures as required to take separation process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

cleaning and sanitising equipment

completing workplace records as required

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the separation process, including stages and changes that occur during the separation process

basic operating principles of equipment for the separation process, including:

safety hazards associated with separation equipment and the implications of interchanging parts/incorrect bowl balance

main equipment components

status and purpose of guards

equipment operating capacities and applications

the purpose and location of sensors and related feedback instrumentation

services required for operation of separation equipment used in the workplace

the flow of the separation process and the effect of product output on downstream processes

impact of in-feed quality and concentration levels on the separation process

quality characteristics and uses of separation process outputs

methods used to monitor the separation process, including inspecting, measuring and testing in-feed and out-feed solids, and other tests as required by the process

inspection or test points (control points) in the separation process and the related procedures and recording requirements

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

common causes of variation and corrective action required, including how variation in temperature and solids affects the process

spoilage and other food safety risks associated with separation, and related control measures

work health and safety hazards and controls, including limitations of protective clothing and equipment relevant to the work process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for operating the separation process

cleaning and sanitation procedures required for operating the separation process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the operation of the separation process, including handling of effluent

basic operating principles of process control where relevant, including the relationship between control panels and systems and the physical equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

separation process and related equipment and services

materials required for the separation process

sampling schedules and test procedures and equipment as required

routine preventative maintenance schedule as required

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements, stock flow systems, production schedules, batch/recipe instructions

information on equipment capacity and operating parameters

production schedule and batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4