# Assessment Requirements for FBPOPR2051 Operate a spreads production process

Release: 1

Assessment Requirements for FBPOPR2051 Operate a spreads production process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a spreads production process on at least one occasion, including:

accessing workplace information to identify processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that related equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting process equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including:

temperature

work input

throughput

pressures

crystal formation

monitoring supply and flow of materials to and from the process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

follow isolation and lock-out/tag-out procedures to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

completing workplace records

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of spreads preparation, including:

crystallisation

relationship between stages of crystal formation, work input and temperature

stages involved in the process

basic operating principles of equipment for the spreads production process, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

services required for the spreads production process and action to take if services are not available

the flow of the spreads production process and the effect of outputs on downstream processes

quality characteristics required of the finished product

quality requirements of oil or emulsion for processing and the effect of variation on process performance

operating requirements and parameters and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes of the spreads production process, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods used to monitor the spreads production process, including inspecting, measuring and testing by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination/food safety risks associated with the process and related control measures

common causes of variation associated with a spreads production process and corrective action required

work health and safety hazards and controls relevant to operating the spreads production process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities for the spreads production process

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the process, including waste/rework collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures relevant to operating the spreads production process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

spread processing and related equipment and services

emulsion or oil to be processed

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule/batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4