# Assessment Requirements for FBPOPR2054 Operate a water purification process

Release: 1

Assessment Requirements for FBPOPR2054 Operate a water purification process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a water purification process on at least one occasion, including:

accessing workplace information to identify water purification process requirements

selecting, fitting and using personal protective clothing and/or equipment

responding appropriately to hazards, including chemical spills

confirming supply of necessary materials and services

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that equipment is clean and correctly configured for water purification process requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting water purification process equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including:

flow rates

pressure

operation of dosing equipment

alarms

monitoring supply and flow of materials to and from the water purification process

taking corrective action in response to out-of-specification results

maintaining a purification system free of physical, chemical and biological contaminants

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures to take water purification process and related equipment off-line in preparation for cleaning/back-flushing and/or maintenance within level of responsibility

carrying out cleaning, sanitising, regenerating and back-flushing

completing workplace records

maintaining work area to meet housekeeping standards

following relevant work health and safety procedures, including responding appropriately to hazards, including chemical spills.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the water purification process, including methods used to purify water appropriate to workplace requirements

basic operating principles of equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

services required and action to take if services are not available

the flow of the water purification process and the effect of outputs on downstream processes

quality characteristics to be achieved by the water purification process

quality requirements of inputs to the purification process and the effect of variation on process performance

operating requirements and parameters, and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including:

following troubleshooting and problem-solving guidelines

recognising signs and symptoms of faulty equipment and early warning signs of potential problems

basic operating principles of process control as appropriate, including the relationship between control panels and systems and the physical equipment

methods used to monitor the water purification process, including inspecting, measuring and testing by the process

inspection or test points (control points) in the water purification process and the related procedures and recording requirements

Good Manufacturing Practice (GMP)/food safety requirements (as appropriate) associated with the purification process and related control measures

common causes of variation and corrective action required

work health and safety hazards and controls

requirements of different shutdowns as appropriate to the water purification process and workplace production requirements, including emergency and routine shutdowns

isolation, lock-out and tag-out procedures and responsibilities

cleaning and sanitation procedures

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the water purification process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

washing and drying equipment and services, including water and sanitiser

raw materials to be processed

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedule/batch instructions

specifications and inspection or control points

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4