# Assessment Requirements for FBPOPR2056 Operate a freezing process

Release: 1

Assessment Requirements for FBPOPR2056 Operate a freezing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a freezing process on at least one occasion, including:

accessing workplace information to identify processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

preparing materials

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

starting, operating, monitoring and adjusting the freezing process and equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including:

tunnel and product temperatures

throughput

removal of frozen waste product from equipment

visually inspecting product

monitoring supply and flow of materials to and from the process

taking corrective action in response to out-of-specification results

correctly following procedures to defrost tunnels

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

completing workplace records

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

cleaning and sanitising equipment according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of freezing, including the changes that occur to product during freezing

basic operating principles of equipment used in the operation of a freezing process, including:

main equipment components

status and purpose of guards

basic principles and operation of refrigeration system

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

the flow of the freezing process and the effect of product output on downstream processes

quality characteristics to be achieved by the freezing process

effect of raw material characteristics on the freezing process

operating requirements and parameters of the freezing process and corrective action required where operation is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

contamination and food safety risks associated with the freezing process and related control measures

common causes of variation in the operation of the freezing process and corrective action required

work health and safety hazards and controls, including the limitations of protective clothing and equipment relevant to the work process

requirements of different shutdowns as appropriate to the process and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of the freezing process

procedures and responsibility for reporting production and performance information relevant to the freezing process

environmental issues and controls relevant to the freezing process, including waste collection and handling procedures

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment

cleaning and sanitation procedures required for the freezing process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

freezing process and related equipment and services

materials to be frozen

sampling schedules and test procedures and equipment

cleaning procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

production schedule and batch instructions

specifications, control points and processing parameters

information on equipment capacity and operating parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4