# Assessment Requirements for FBPOPR2057 Operate a membrane process

Release: 1

Assessment Requirements for FBPOPR2057 Operate a membrane process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has operated a membrane process on at least one occasion, including:

accessing workplace information to identify membrane processing requirements

selecting, fitting and using personal protective clothing and/or equipment

confirming supply of necessary materials and services

conducting pre-start check, including:

inspecting equipment/membrane condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock-outs

confirming that equipment, including pumps, valves and pipework, are clean and correctly configured for membrane processing requirements

sensors and controls are correctly positioned

any scheduled maintenance has been carried out

all safety guards are in place and operational

starting, operating, monitoring and adjusting membrane processing equipment to achieve required outcomes, including monitoring control points and conducting inspections to confirm process remains within specification, including monitoring:

temperatures

flow rates

total solids

pressure/vacuum

monitoring supply and flow of materials to and from the membrane process

taking corrective action in response to out-of-specification results

responding to and/or reporting equipment failure within level of responsibility

locating emergency stop functions on equipment

following isolation and lock-out/tag-out procedures to take membrane processing and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility

following procedures to clean membranes

completing workplace records

maintaining work area to meet housekeeping standards

using process control systems according to workplace procedures

following relevant work health and safety procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of membrane processing, including:

components of milk

particle size of each component

effect on these components of membrane processing

methods used to achieve this outcome

basic operating principles of membrane processing equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

material composition of membranes used in the workplace, and related operating, cleaning and storage requirements

the flow of the membrane process, including the different processing streams and the effect of the membrane process outputs on downstream processes

quality characteristics required of the membrane processing outputs

the effect of variation in raw material characteristics on membrane processing performance

operating requirements and parameters and corrective action required where operation is outside specified operating parameters, including the effect of variation in processing parameters, including temperature, pressure, total solids and flow rates on membrane process performance

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

techniques used to monitor membrane processing, including inspecting, measuring and testing by the process

inspection or test points (control points) in the process and the related procedures and recording requirements

contamination and food safety risks associated with the process and related control measures

common causes of variation and corrective action required when operating a membrane process

work health and safety hazards and controls, including the limitations of protective clothing and equipment relevant to the work process

requirements of different shutdowns as appropriate to membrane processing and workplace production requirements, including emergency and routine shutdowns, and procedures to follow in the event of a power outage and shutdown

cleaning and sanitation requirements required for operating a membrane process

isolation, lock-out and tag-out procedures and responsibilities

procedures and responsibility for reporting membrane processing production and performance information

environmental issues and controls relevant to membrane processing, including waste collection and handling procedures related to the process

basic operating principles of process control, including the relationship between control panels and systems and the physical equipment.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

membrane processing and related equipment and services

materials required for membrane processing

sampling schedules and test procedures and equipment

cleaning and sanitation procedures, materials and equipment

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements, stock flow systems, production schedules, batch/recipe instructions

information on equipment capacity and operating parameters

production schedule and batch instructions

specifications, control points and processing parameters

documentation and recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4