# Assessment Requirements for FBPOPR2092 Operate a continuous freezing process

Release: 1

Assessment Requirements for FBPOPR2092 Operate a continuous freezing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated a continuous freezing process to freeze at least one batch of product to meet specifications, including:

applying safe work procedures

following food safety practices

monitoring control points and conducting inspections to confirm process remains within specification, including:

flow rates

churn speed

time/temperature

ingredient addition systems

pressures

equipment condition and performance

addressing routine processing issues.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the continuous freezing process, including the stages that occur during the churning and freezing process, and the required physical and microstructure of the product

basic operating principles of equipment required to operate a continuous freezing process, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

services required to operate a continuous freezing process, and action to take if services are not available

the flow of the freezing process and the effect of outputs on downstream processes

additives and ingredients used in the continuous freezing process, and their role in the product

quality characteristics to be achieved by the freezing process

quality requirements of materials/ingredients, and effect of variation on process performance

operating requirements and parameters, and corrective action required where operation of the continuous freezing process is outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment, and early warning signs of potential problems

methods used to monitor the production process, including inspecting, measuring and testing by the process

inspection or test points (control points) in the continuous freezing process operation and the related procedures and recording requirements

contamination/cross-contamination and food safety risks associated with the continuous freezing process, and related control measures

common causes of variation in the operation of a continuous freezing process, and corrective action required

health and safety hazards and controls relevant to the operation of a continuous freezing process, including the limitations of protective clothing and equipment relevant to the work process

requirements of different shutdowns as appropriate to the continuous freezing process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

isolation, lock-out and tag-out procedures and responsibilities required for the operation of a continuous freezing process

procedures and responsibility for reporting production and performance information of the continuous freezing process

environmental issues and controls relevant to the continuous freezing process, including waste/rework collection and handling procedures related to the process

basic operating principles of continuous freezing process control, including the relationship between control panels and systems and the physical equipment

cleaning procedures required for the continuous freezing equipment

recording procedures to ensure traceability.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a food processing workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing

freezing process and related equipment and services

materials/ingredients to be frozen

specifications:

work procedures, including advice on safe work practices, food safety, quality and environmental requirements

information on equipment capacity and operating parameters

production schedules and batch specifications, control points and processing parameters

recording requirements and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4