# FBPOPR3008 Load and unload tankers

Release: 1

FBPOPR3008 Load and unload tankers

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to prepare, load and unload transport tankers with bulk product, and maintain records of product.

This unit applies to individuals who have responsibility for loading and unloading tankers and who resolve routine problems with that process.

All work must be carried out to comply with workplace procedures according to state/territory health and safety, environmental regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Operational (OPR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare transport tankers for loading | 1.1 Confirm work instructions and clarify where required 1.2 Identify health, safety and food safety hazards, and confirm controls are in place 1.3 Wear appropriate personal protective equipment and clothing 1.4 Check the loading schedule to confirm loading requirements 1.5 Confirm that transfer equipment is in operational condition 1.6 Select transfer equipment settings and operating parameters to meet safety and loading requirements 1.7 Locate bulk materials to confirm type, quantity available and clearance for loading 1.8 Confirm priorities for loading to meet delivery requirements |
| 2. Check product for loading | 2.1 Check that receival record confirms product type and quantities 2.2 Clear product for loading according to workplace procedures and safety requirements 2.3 Identify bulk storage locations, holdings and capacities 2.4 Confirm actual-to-recorded holding levels 2.5 Confirm transfer equipment is in operational condition 2.6 Confirm priorities for loading to meet delivery requirements |
| 3. Operate and monitor the tanker loading and unloading process | 3.1 Start and operate the tanker loading process according to safe work procedures 3.2 Monitor equipment to confirm load transfer to tanker in correct quantities 3.3 Monitor equipment to ensure optimal operations 3.4 Take samples of product for testing 3.5 Select equipment settings to complete loading operation 3.6 Purge in-flight product from lines 3.7 Clean equipment and prepare for further loading according to safe work and quality standards 3.8 Record information on the loading operation according to workplace procedures |
| 4. Unload product and complete housekeeping | 4.1 Start and operate the tanker unloading process according to safe work procedures 4.2 Select equipment settings to complete unloading operation 4.3 Purge in-flight product from lines 4.4 Clean and shut down equipment and prepare for further work in line with workplace procedures 4.5 Ensure waste is disposed of in line with environmental requirements 4.6 Record information on the unloading operation according to workplace procedures 4.7 Identify and report maintenance requirements according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Learning | Resolve routine problems with pumping equipment |
| Reading | Interpret information relevant to product and loading requirements |
| Writing | Record data and observations using digital and/or paper-based formats |
| Numeracy | Interpret capacities (m3), quantities (mL, L, hL), temperature (C) |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOPR3008 Load and unload tankers | FBPOPR2009 Load and unload tankers | Unit code updated to better match complexity of work task Minor changes to Performance Criteria to clarify intent Foundation Skills refined Performance Evidence clarified Minor changes to Knowledge Evidence and Assessment Conditions | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPOPR3008 Load and unload tankers

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has loaded and unloaded at least one load of product into and out of a tanker, including:

applying safe work practices

applying quality standards to work practices

completing records accurately.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

bulk storage facility layout and related management systems

basic operating principles of the system and equipment used, including the features of the tanker loading/unloading and materials transfer systems

equipment that controls the flow and transfer of bulk materials, equipment operating capacity and the status and purpose of guards

tanker types and capacities

planning and scheduling systems and responsibilities to prioritise loading and unloading processes, including the characteristics of materials received, and related storage requirements, production requirements and storage capacities

signs and symptoms of faulty equipment and early warning signs of potential problems, including leaks, unusual noises and/or vibrations in pumps

operating parameters, and corrective action required where operation is outside specified operating parameters

product inspection and procedures within level of responsibility

steps in preparing transfer equipment for use, including confirming that equipment is clean, lines are purged, isolation or lock-outs are cancelled as required, and any scheduled maintenance has been completed

steps in confirming that tankers are in suitable condition for loading, including checking cleaning records and confirming that any scheduled maintenance has been carried out

steps in confirming that correct quantities are transferred at the required times, to the required locations, including monitoring pump loads, flow rates and filter/strainer condition

clearance procedures, including relevant test methods, and action required if results are not within specification or clearance is not provided

health and safety hazards and controls, including emergency procedures to respond to hazardous situations

contamination risks and controls, including cross-contamination risks, product compatibility and storage requirements

awareness of common microbiological, physical, chemical and allergen hazards related to the foods and/or products handled in the work area, including the types of hazards likely to occur, the conditions under which they occur, possible consequences, and control methods to prevent occurrence

when and how to use emergency stop functions on equipment

isolation, lock-out and tag-out procedures and responsibilities

environmental issues associated with bulk liquid materials transfer, including consequences of spills and appropriate control/containment procedures

recording requirements and procedures, including related receivals and dispatch documentation

cleaning procedures for loading and unloading equipment and tanker

procedures to record traceability of product and ingredients.

# Assessment Conditions

Assessment of the skills in this unit of competency must take place under the following conditions:

physical conditions:

a processing workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective clothing and equipment

loading/unloading schedules and related receivals and/or dispatch documentation

tankers, storage facilities and materials transfer equipment

product for loading/unloading

cleaning materials and equipment

specifications:

work procedures, including advice on safe work practices, quality and environmental requirements

information about equipment/tanker capacity and operating parameters

loading/unloading information recording system and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4