# FBPOPR3008 Load and unload tankers

Release: 1

FBPOPR3008 Load and unload tankers

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 6.0. |

# Application

This unit of competency describes the skills and knowledge required to prepare, load and unload transport tankers with bulk product, and maintain records of product.

This unit applies to individuals who have responsibility for loading and unloading tankers and who resolve routine problems with that process.

All work must be carried out to comply with workplace procedures according to state/territory health and safety, environmental regulations, legislation and standards that apply to the workplace.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Operational (OPR)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare transport tankers for loading | 1.1 Confirm work instructions and clarify where required 1.2 Identify health, safety and food safety hazards, and confirm controls are in place 1.3 Wear appropriate personal protective equipment and clothing 1.4 Check the loading schedule to confirm loading requirements 1.5 Confirm that transfer equipment is in operational condition 1.6 Select transfer equipment settings and operating parameters to meet safety and loading requirements 1.7 Locate bulk materials to confirm type, quantity available and clearance for loading 1.8 Confirm priorities for loading to meet delivery requirements |
| 2. Check product for loading | 2.1 Check that receival record confirms product type and quantities 2.2 Clear product for loading according to workplace procedures and safety requirements 2.3 Identify bulk storage locations, holdings and capacities 2.4 Confirm actual-to-recorded holding levels 2.5 Confirm transfer equipment is in operational condition 2.6 Confirm priorities for loading to meet delivery requirements |
| 3. Operate and monitor the tanker loading and unloading process | 3.1 Start and operate the tanker loading process according to safe work procedures 3.2 Monitor equipment to confirm load transfer to tanker in correct quantities 3.3 Monitor equipment to ensure optimal operations 3.4 Take samples of product for testing 3.5 Select equipment settings to complete loading operation 3.6 Purge in-flight product from lines 3.7 Clean equipment and prepare for further loading according to safe work and quality standards 3.8 Record information on the loading operation according to workplace procedures |
| 4. Unload product and complete housekeeping | 4.1 Start and operate the tanker unloading process according to safe work procedures 4.2 Select equipment settings to complete unloading operation 4.3 Purge in-flight product from lines 4.4 Clean and shut down equipment and prepare for further work in line with workplace procedures 4.5 Ensure waste is disposed of in line with environmental requirements 4.6 Record information on the unloading operation according to workplace procedures 4.7 Identify and report maintenance requirements according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Learning | Resolve routine problems with pumping equipment |
| Reading | Interpret information relevant to product and loading requirements |
| Writing | Record data and observations using digital and/or paper-based formats |
| Numeracy | Interpret capacities (m3), quantities (mL, L, hL), temperature (C) |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPOPR3008 Load and unload tankers | FBPOPR2009 Load and unload tankers | Unit code updated to better match complexity of work task Minor changes to Performance Criteria to clarify intent Foundation Skills refined Performance Evidence clarified Minor changes to Knowledge Evidence and Assessment Conditions | Equivalent |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4