# Assessment Requirements for FBPPBK2005 Operate a doughnut making process

Release: 1

Assessment Requirements for FBPPBK2005 Operate a doughnut making process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 1.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has set up, operated, adjusted and shut down a doughnut-making process on at least three occasions, including:

accessing workplace information to identify doughnut manufacturing requirements

selecting, fitting and using personal protective equipment

confirming supply of necessary materials and services

confirming condition of materials, including confirming flour and water temperature, flour type and oil type

conducting pre-start checks, including:

inspecting equipment condition to identify any signs of wear

selecting appropriate settings and/or related parameters

cancelling isolation or lock outs

confirming that equipment is clean and correctly configured for processing requirements

positioning sensors and controls correctly

ensuring any scheduled maintenance has been carried out

confirming that all safety guards are in place and operational

conducting pre-start checks for equipment, including:

doughnut mixing and forming equipment

deep frying equipment

cooling tunnels/spirals

starting, operating, monitoring and adjusting process equipment to achieve required outcomes, including monitoring:

materials transfer prior to commencing mixing

mixing parameters for water and flour temperature

sequence of ingredient addition and mixing times

forming parameters, including air pressure to maintain weight of doughnut

proving parameters - humidity and temperature within prover and time

frying parameters -  oil temperature, appearance of cooked doughnuts and time

operating finishing/filling equipment according to workplace procedures, including:

preparing decorating ingredients

mixing and heating, crushing, cutting or sieving toppings

monitoring temperature of toppings, spread/application of toppings and glazes, jam/filling placement and quantity and appearance of finished product

using process control systems according to workplace procedures

taking corrective action in response to out-of-specification results in line with own role and responsibilities

following work health and safety procedures including:

responding to and/or reporting equipment failure

locating emergency stop functions on equipment

following isolation and lock out or tag out procedures to take process and related equipment off-line in preparation for cleaning and/or maintenance

completing workplace records

applying environmental and food safety policies and procedures to work practices, including:

maintaining work area to meet housekeeping standards

cleaning and sanitising equipment according to workplace procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of the doughnut manufacturing process

stages and requirements of each processing stage in doughnut manufacture, including:

mixing

proving

frying

filling

finishing

role of main ingredients, including:

flour

yeast

water

ice

oil

glazes

topping

pre-mixes

types of equipment, including:

doughnut mixing and forming equipment

filling equipment

finishing equipment

deep frying equipment, gas and electric

cooling tunnels and spirals

weigh scales

hoppers

forming equipment

cutters

injectors

provers

enrobers

basic operating principles of equipment, including:

main equipment components

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

starting, operating, monitoring and adjusting process equipment to achieve required outcomes

typical equipment faults and related causes, including signs and symptoms of faulty equipment, loss of power, and early warning signs of potential problems

basic operating principles of process control, including the relationship between control panels and systems and the doughnut production equipment

key features of the flow of the doughnut manufacturing process and the effect of outputs on customer satisfaction and downstream processes, including packaging

quality requirements of materials and effect of variation on doughnut manufacturing process performance

quality characteristics to be achieved by the finished doughnut

process specifications, procedures and operating parameters, including procedures for adjusting water temperature to compensate for atmospheric conditions

operating requirements and parameters and corrective action required where results are out-of-specification operating parameters

methods used to monitor the doughnut manufacturing process, including:

inspecting, measuring and testing

inspection or test points (control points) in the process

related procedures and recording requirements

temperature of toppings

spread and application of toppings and glazes

placement of jams and fillings

quantity and appearance of finished product

common causes of variation and corrective action required

doughnut-finishing requirements, including:

preparation methods

monitoring of parameters

application of finishings

quality requirements

work health and safety hazards and controls, including:

limitations of personal protective equipment

routine shut down requirements

emergency shut down procedures

procedures to follow in the event of a power outage

isolation, lock out and tag out procedures and responsibilities

procedures and responsibility for reporting production and performance information

environmental and food safety issues and controls relevant to the doughnut manufacturing process, including:

waste and rework collection and handling

contamination and food safety risks

cleaning and sanitation procedures.

# Assessment Conditions

Assessment of this unit of competency must take place under the following conditions:

physical conditions:

a workplace or an environment that accurately represents workplace conditions

resources, equipment and material:

personal protective clothing and equipment

doughnut mixing, forming, proving, injecting and frying equipment, and glazing and topping application equipment

doughnut ingredients, frying oil and glazing, or toppings and fillings to meet production requirement

documentation or technology for recording information

cleaning procedures, materials and equipment

specifications:

work instructions including control points and doughnut production processing parameters

production schedule and recipe and batch specifications for doughnut products

information on equipment capacity and operating parameters

workplace procedures for doughnut production process

relationships (internal and/or external):

workplace supervisor

timeframes:

according to work requirements.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4