# FBPPBK3001 Operate a dough mixing process

Release: 1

FBPPBK3001 Operate a dough mixing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0. |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a dough mixing process.

This unit applies to individuals who apply basic operating principles to the operation and monitoring of a dough mixing process in a production baking environment.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory work health and safety, and food safety regulations, legislation and standards that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication

# Pre-requisite Unit

Nil

# Unit Sector

Production baking (PBK)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare dough making equipment and process for operation | 1.1 Identify and fit personal protective equipment required for operation 1.2 Identify and confirm cleaning and maintenance status 1.3 Weigh or measure ingredients to meet recipe requirements 1.4 Fit and adjust mixing equipment in accordance with operating requirements 1.5 Enter processing and operating parameters to meet safety and production requirements 1.6 Check and adjust equipment performance as required 1.7 Carry out pre-start checks as required by workplace procedures |
| 2. Operate and monitor dough mixing process | 2.1 Start up the dough mixing process according to workplace procedures 2.2 Introduce ingredients to mixing process in correct sequence and quantity 2.3 Monitor equipment and identify and report variations in operating conditions 2.4 Monitor process to confirm dough meets specifications 2.5 Identify and rectify or report outcomes that fail to meet product specifications 2.6 Transfer finished dough according to workplace procedures 2.7 Maintain work area and conduct activities according to workplace procedures, food safety and environmental guidelines 2.8 Maintain workplace records in required format |
| 3. Shut down the process | 3.1 Identify appropriate shutdown procedure 3.2 Shut down process safely and clean equipment according to workplace procedures 3.3 Identify and report maintenance issues according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interpret standard operating procedures and directions for the safe operation of equipment when operating the dough mixing process |
| Writing | Record maintenance requirements Record equipment failure |
| Numeracy | Confirm settings including mixing rate, mixing time, water and flour temperature Monitor supply and flow of materials to and from the dough making process |
| Navigate the world of work | Apply workplace procedures to own role and responsibilities and seek clarification or other assistance when required Maintain hygiene standards and hazard free work area |
| Interact with others | Report dough making process performance to relevant personnel using required communication method |
| Get the work done | Identify routine problems and implement standard solutions according to workplace guidelines |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPPBK3001 Operate a dough mixing process | FDFPB3001A Operate a dough mixing process | Updated to meet Standards for Training Packages | Equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPPBK3001 Operate a dough mixing process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has demonstrated all steps of the dough mixing process at least once, including:

identifying and acquiring ingredients and personal protective equipment required for the process

safely conducting pre-start checks on process equipment

loading ingredients in correct quantities and sequences for the recipe and the process

starting, operating, monitoring and adjusting process equipment to ensure that product meets specifications

taking corrective action in response to results that fail to meet specifications

completing required workplace documentation

safely shutting down process and cleaning equipment according to workplace requirements.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and basic principles of dough-making process, including components of wheat, flour types and grades relevant to products produced, yeast activity and dough development

effects of variation in ingredient quality on the dough produced

basic operating principles of equipment including equipment components, status and purpose of guards, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation

settings relevant to dough making equipment being used, including:

mixing rate

mixing time and work input

ice and water requirements

water temperature

speed

flour temperature

cooling

vacuum delay and level

slurry addition

flow of the dough-making process, including the stages and changes which occur during baking, and the effect of outputs on downstream processes

significance of factors, such as dough temperature, development, time, water addition and temperature, yeast activity

required characteristics of dough and effect of dough characteristics and variation on the final product

operating requirements and parameters and corrective action required where operation is outside specified parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems, and procedures and responsibilities for addressing issues

methods used to monitor the dough making process, including as inspecting, measuring and testing

health and food safety risks associated with the dough making process and related control measures

requirements of different shutdowns as appropriate to the dough making process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

procedures and responsibility for reporting production and performance information

workplace cleaning procedures for the dough making process

environmental issues and controls relevant to the dough making process, including waste collection and handling procedures.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace setting or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment

dough mixing process and related equipment and services

ingredients to be processed

cleaning materials and equipment

workplace record keeping system

specifications:

work procedures, including advice on safe work practices, cleaning, food safety, quality, cleaning and environmental requirements

manuals related to operation of dough making equipment

dough recipe(s).

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4