# Assessment Requirements for FBPPBK3003 Operate a dough make up process

Release: 1

Assessment Requirements for FBPPBK3003 Operate a dough make up process

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has, on at least occasion, demonstrated all steps of the dough make up process, including:

confirming dough make up processing requirements

safely conducting pre-start checks of the dough make up process

confirming supply of oiling pans, setting pan stops and guides for panning unit

safely starting, operating, monitoring and adjusting process equipment and conducted inspections to confirm process conforms to specifications including:

dough height in hopper

dough weight

oil flow to divider

size and shape of dough piece

skin formation during intermediate proof

presence of ‘doubles’

process variables such as time, temperature and humidity

taking corrective action in response to results that fail to meet specifications

completing required workplace documentation

complying with food safety requirements

safely shutting down process and cleaning equipment according to workplace requirements.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

the purpose and basic principles of each stage of the dough make up process

basic operating principles of equipment including main equipment components, status and purpose of guards, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation

flow of the dough make up process and the effect of outputs on downstream processes

effect of process variables including time, temperature and humidity on process stages

procedures for carrying out pre-start checks, including:

divider speed, weight setting and oil supply

air flow from fans in rounder

intermediate proving process airflow

moulding process settings, including sheeting, sealing board height and width, four piecing and roller adjustments

panning process settings, including side guides, sealing board, knives, folders and side belts

effect of process stages including moulding on dough structure

quality characteristics to be achieved by the dough make up process

quality requirements of materials and effects of variation on the dough make up process

operating requirements and parameters and corrective action required if operation falls outside specified operating parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

methods of monitoring dough make up process, such as inspecting, measuring and testing as required

inspection or test points (control points) in dough make up process and the related procedures and requirements

contamination and food safety risks associated with the dough make up process and related control measures

possible work health and safety hazards and controls relevant to the work process, including the limitations of personal protective equipment

requirements of different shutdowns and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage

cleaning methods and procedures

isolation, lock out or tag out procedures and responsibilities

cleaning and sanitation procedures

procedures and responsibility for reporting production and performance information

environmental issues and controls relevant to the dough make up process, including waste collection and handling procedures related to the process.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a workplace setting or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment

dough make up and transfer equipment and services

dough to be processed

workplace record keeping system

specifications:

work procedures, including advice on safe work practices, food safety, quality, cleaning and environmental requirements

information on equipment capacity and operating parameters

production schedule and batch instructions

sampling schedules and test procedures and equipment as required

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4