# FBPPHM3016 Operate a sterilisation process using an autoclave

Release: 1

FBPPHM3016 Operate a sterilisation process using an autoclave

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Application

This unit of competency describes the skills and knowledge required to set up, operate, adjust and shut down a sterilisation process using an autoclave in a pharmaceutical manufacturing facility.

The unit applies to individuals who apply Good Manufacturing Practice (GMP) and operating principles to the sterilisation process using an autoclave. Individuals work under broad direction and take responsibility for their own work.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Pharmaceutical (PHM)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare equipment and process for operation | 1.1 Confirm autoclave and wrapping, cleaning and material items are available and meet load pattern requirements 1.2 Prepare autoclave and position load probe according to load pattern requirements and standard operating procedure 1.3 Enter operating parameters according to safety and sterilisation requirements 1.4 Conduct pre-start checks according to workplace requirements 1.5 Load wrapping, cleaning and material items according to load pattern requirements |
| 2. Operate autoclave to terminally sterilise items | 2.1 Start up and monitor autoclave to confirm specifications are within required limits 2.2 Identify and report out of limit materials or processes according to workplace procedures 2.3 Maintain work area according to workplace cleaning standards 2.4 Conduct process according to safety and environmental requirements 2.5 Complete documentation according to workplace procedures |
| 3. Remove items and follow shut down procedures | 3.1 Confirm the workplace procedures for shutting down the process 3.2 Safely shut down the process 3.3 Remove items from autoclave 3.4 Check chemical indication to confirm items are dry 3.5 Ensure segregation of sterile and non-sterile items 3.6 Complete records according to workplace procedures |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Identify relevant information from workplace documentation and interpret requirements for the sterilisation process |
| Writing | Complete workplace documentation using appropriate language and in required format |
| Numeracy | Confirm number of items according to loading pattern Interpret time and temperature specifications for cycle parameters Confirm post cycle sampling results meet sterilisation requirements |
| Navigate the world of work | Identify workplace procedures relevant to own role |
| Interact with others | Report operational and safety information to relevant personnel using required communication method |
| Get the work done | Plan, organise and implement tasks required to achieve sterilisation process outcomes Use problem-solving skills to analyse product and process faults and decide on appropriate action |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Pre-start checks must include: | carrying out required area or line clearances inspecting equipment condition to identify signs of wear confirming all safety equipment is in place and operational confirming that equipment is clean or sanitised confirming that equipment is correctly configured for processing requirements. |
| --- | --- |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPPHM3016 Operate a sterilisation process using an autoclave | FDFPH2014A Operate a terminal sterilisation process | Updated to meet Standards for Training Packages Changes to performance criteria for clarity Code changed to reflect AQF alignment | Equivalent unit |

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPPHM3016 Operate a sterilisation process using an autoclave

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all the elements and performance criteria of this unit.

There must be evidence that the individual has operated at least one sterilisation process using an autoclave, including:

accessed workplace information to confirm sterilisation requirements

confirmed supply of necessary wrapping, cleaning and material items for the sterilisation process

prepared items by wrapping and marking with temperature-sensitive tape or material and ensured clothing is folded correctly

positioned load probe and loaded items in correct orientation according to workplace procedures and load pattern requirements

conducted pre-start checks required for safe operation of the autoclave, including:

carrying out required area or line clearances

inspecting equipment condition to identify signs of wear

confirming all safety equipment is in place and operational

confirming that equipment is clean or sanitised

confirming that equipment is correctly configured for processing requirements

started, operated, monitored and adjusted autoclave to achieve required outcomes, including time and temperature parameters

conducted in-process control checks to confirm the process remains within limits

collected samples and conducted tests according to workplace procedures

taken corrective action in response to a non-conformance

opened autoclave and removed items from autoclave according to workplace procedures

used process control systems according to workplace procedures

safely shut down the process according to workplace procedures

cleaned and maintained work area to meet workplace cleaning standards and environmental requirements

completed records according to workplace procedures.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

stages of the sterilisation process, including:

principles of heat transfer

properties of saturated steam

the relationship between time, temperature and pressure as they affect the kill rate

variables that impact the effectiveness of sterilisation

the nature of items to be sterilised using an autoclave and related handling and preparation requirements, including:

porous loads

non-porous loads

position of probes

the nature of wrapping materials suitable for use, including:

methods used to wrap items

methods used to monitor whether time and temperature parameters are met (including heat-sensitive tape or markers)

the significance of loading patterns in achieving effective sterilisation

basic operating principles of equipment, requirements and parameters of autoclave, including:

main equipment components

time and temperature parameters required to achieve sterilisation and corrective actions required where operation falls outside specified parameters

typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems

status and purpose of guards

equipment operating capacities and applications

purpose and location of sensors and related feedback instrumentation

corrective actions required where operation is outside of specified parameters

calibration schedule and procedures to confirm that instruments are within calibration

health and safety hazards related to autoclave operation and associated control measures, including working with superheated steam and hot surfaces, manual handling and steam leaks

pre-start checks requirements, including:

carrying out required area or line clearances

inspecting equipment condition to identify signs of wear

confirming all safety equipment is in place and operational

confirming that equipment is clean or sanitised

confirming that equipment is correctly configured for processing requirements

evaluation and in-process monitoring of sterilisation process indicators, including:

biological

chemical

physical

methods used to monitor the sterilisation process, including:

inspecting

measuring

testing

requirements of different shutdowns, including:

emergency and routine shutdowns

procedures to follow in the event of a power outage

isolation, lock out and tag out procedures and responsibilities

operating principles of process control

Good Manufacturing Practice (GMP) requirements associated with a sterilisation process and related control measures

environmental issues and controls relevant to the sterilisation process including waste collection and handling procedures

requirements for completion of workplace documentation.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a pharmaceutical manufacturing workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

autoclave and handling equipment

wrapping materials and tape/heat sensitive markers

materials for sterilisation

test equipment

cleaning materials and equipment associated with sterilisation process

record keeping system

specifications:

sterilisation specifications, control points and processing parameters

recording requirements and procedures

workplace documentation relating to sterilisation process and procedures that comply with GMP requirements

load pattern requirements

sampling schedules and test procedures

cleaning procedures associated with sterilisation process.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4