# FBPRBK3005 Produce basic bread products

Release: 1

FBPRBK3005 Produce basic bread products

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0 |

# Application

This unit of competency describes the skills and knowledge required to produce basic bread products in a commercial baking environment.

This unit applies to individuals who apply a broad range of knowledge and skills with responsibility for their own work. This includes applying and communicating known solutions to predictable problems.

All work must be carried out to comply with workplace procedures, in accordance with State/Territory food safety, and work health and safety, regulations and legislation that apply to the workplace.

No occupational licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Retail baking (RBK)

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Prepare for work | 1.1 Confirm product types and volumes to schedule production 1.2 Calculate yield and adjust recipe to meet required production volumes 1.3 Confirm work area and work practices meet food safety and workplace health and safety requirements 1.4 Select and wear personal protective equipment according to safety requirements 1.5 Select basic bread baking equipment and check to confirm readiness for use 1.6 Select ingredients and check to confirm quality and quantity |
| 2. Mix basic bread dough | 2.1 Measure ingredient quantities to meet recipe specifications 2.2 Load ingredients into the mixer in the required ingredient placement 2.3 Operate and monitor mixer to achieve basic bread dough development for product type 2.4 Check mixed basic dough to identify faults and rectify |
| 3. Process basic bread dough | 3.1 Divide, scale, mould and intermediate prove basic dough to meet required end-product shape and baked weight 3.2 Final mould basic dough and place on baking surfaces and pre-prove score for product type before final prove 3.3 Retard basic dough as required for product type 3.4 Final prove basic dough as required for product type 3.5 Check processed basic dough to identify faults and rectify |
| 4. Pre-bake finish basic bread products | 4.1 Prepare finishing ingredients to meet recipe specifications 4.2 Pre-bake finish basic bread products to meet end-product specifications 4.3 Check pre-bake finished basic bread products to identify faults and rectify |
| 5. Bake basic bread products | 5.1 Set baking temperatures and times to prepare for baking 5.2 Visually check basic dough size to confirm readiness for baking, and post-prove score for product type prior to baking 5.3 Load oven and steam as required for product type 5.4 Monitor baking to achieve baked colour and stability required for basic bread product type 5.5 Unload and de-pan baked basic bread products to cool 5.6 Check basic bread product bake to identify faults and rectify 5.7 Prepare and transfer products for presentation and storage in accordance with packaging and food safety requirements |
| 6. Complete work | 6.1 Clean equipment and work area to meet housekeeping standards 6.2 Dispose of waste according to workplace requirements 6.3 Complete workplace records according to workplace requirements |

# Foundation Skills

This section describes those language, literacy, numeracy and employment skills that are essential for performance in this unit of competency but are not explicit in the performance criteria.

| Skill | Description |
| --- | --- |
| Reading | Interprets key information from recipes, ingredient labels, baking equipment operating instructions and end-product specifications |
| Writing | Prepares production schedules and completes production records using required format, language and structure |
| Numeracy | Identifies and comprehends ingredient quantities in recipes, dates on ingredient labels, and temperature, humidity and timer settings Estimates approximate quantities, and uses equipment to measure ingredient weights and volumes, and water and dough temperatures Divides dough into equal portions by estimated weight Performs calculations to adjust recipes using baking formulas, and allows for wastage Uses understanding of three-dimensional shapes to mould dough shapes and check end-product shapes Interprets measurement information to set, monitor and adjust process parameters Completes production records using mathematical symbols and conventions |
| Navigate the world of work | Complies with workplace health and safety and food safety regulatory requirements relevant to a commercial baking environment Follows organisational policies and procedures relevant to own work role |
| Get the work done | Plans, organises and implements tasks required to achieve production outcomes Uses problem-solving skills to analyse product and process faults and decide on appropriate action |

# Range of Conditions

This section specifies different work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included.

| Basic bread baking equipment must include: | equipment, including: industrial oven industrial mixer and attachments industrial dough moulding equipment equipment accessories, including: industrial dough moulding equipment accessories ancillary equipment, including: dry ingredients storage containers industrial baking tins and lids bread baking trays ingredient storage bins tools and utensils, including: dough knives dough scrapers water spray bottles scoring knives flour sieves. |
| --- | --- |

# Unit Mapping Information

| Code and title current version | Code and title previous version | Comments | Equivalence status |
| --- | --- | --- | --- |
| FBPRBK3005 Produce basic bread products | FDFRB3002A Produce bread dough | Redesigned unit that includes content from previous unit | No equivalent unit |
|  | FDFRB3005A Bake bread | Redesigned unit that includes content from previous unit | No equivalent unit |
|  | FDFRB3009A Retard dough | Redesigned unit that includes content from previous unit | No equivalent unit |
|  | FDFRB3010A Process dough | Redesigned unit that includes content from previous unit | No equivalent unit |
|  | FDFRB3011A Diagnose and respond to product and process faults (bread) | Redesigned unit that includes content from previous unit | No equivalent unit |

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4

Assessment Requirements for FBPRBK3005 Produce basic bread products

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has produced basic bread products according to a production schedule, including:

mixing and using the following three dough types:

premix dough

instant dough/no time dough/scratch mix dough

chemical leavened dough

producing the following two basic bread types:

white bread

brown bread

producing the following six bread product varieties and shapes:

condensed/sandwich loaves

high top loaves, either married or single

soft/enriched bread roll variety

crusty/lean bread roll variety

crusty/lean loaf – plaited, Vienna, French stick or cob

chemical leavened dough

retarding one of the following three product types produced:

soft/enriched bread roll variety

crusty/lean bread roll variety

crusty/lean loaf – plaited, Vienna, French stick or cob

selecting and using tins and trays appropriate for dough piece scale weight, volume and size

using the following eight moulding techniques:

rounding

batons

knots

4 piecing

degassing

curling/rolling

sealing

cutting/portioning

using the following four bread dough finishing techniques:

pre-prove scoring

post-prove scoring

seeding

dusting

using the following five baking techniques:

steaming

testing bake by sight and baked colour

de-panning basic breads

cooling basic bread products on cooling wires

using lids to condense loaves

selecting, using and cleaning the bread baking equipment listed in the range of conditions, including controlling electrical hazards applicable to cleaning

documenting the production schedule, including the following six considerations:

timings

volume requirements

product processing requirements

recipe reformulation to minimise waste

finishing parameters for basic bread products

bake parameters for basic bread products

Wastage that cannot be reused in production must be less than 5% of the weight of the original production schedule.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

workplace health and safety requirements, including personal protective equipment, applicable to producing basic bread products

safe use and cleaning of bread baking equipment listed in the range of conditions, including electrical hazards

regulatory requirements applicable to producing basic bread products

techniques and considerations for production scheduling, including:

timings

volume requirements

product processing requirements

recipe reformulation to minimise waste

finishing parameters for basic bread products

bake parameters for basic bread products

functions and characteristics of ingredients used in basic bread production, including:

premixes

baker’s or strong flour

whole meal flour

baking powder

salt

sugar

yeast, including principles of fermentation

water

shortening, fats and oils

bread improvers

mixing processes and gluten development for basic bread doughs, including:

premix doughs

scratch/no time/instant doughs

chemical leavened doughs

sequencing of the rapid or no time basic bread production processes

purpose and techniques for pre-bake finishing basic breads

characteristics and techniques for moulding basic bread doughs

impacts of factors on production of different retarded basic bread products, including:

finished dough temperature (FDT)

dough size

dough shape

yeast activity

processing retard in cool phase

processing retard in warm phase

techniques for calculating yields, adjusting recipes, converting units of measurement and measuring ingredients

techniques for selecting tins and trays appropriate for dough piece scale weight, volume and size

required baked characteristics of basic bread products, including:

volume

grain

texture

crumb colour

crust formation

crust colour

the effects of common bread faults in basic bread production, including:

lack of salt

too much salt

no improver

too much improver

no yeast

too much yeast

weak flour instead of strong flour

techniques for disposing of waste from bread production

basic bread production terminology.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a commercial bakery or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment

equipment specified in the range of conditions

bread slicer

ingredients that meet the basic bread product types required in the performance evidence

specifications:

recipes that meet the basic bread product types required in the performance evidence

end-product quality specifications to meet the basic bread product types required in the performance evidence

food safety code regulatory requirements applicable to producing basic bread products

timeframes:

according to the production schedule.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4