# Assessment Requirements for FBPRBK3008 Produce sponge cake products

Release: 1

Assessment Requirements for FBPRBK3008 Produce sponge cake products

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has produced sponge cake products according to a production schedule, including:

producing the following four types of sponge products:

decorated sponge cake (round or square), minimum 7-inch or 18-centimetre diameter in size

sponge roll

individual portioned sponge product

sponge layer cake

mixing and using the following two sponge cake batters:

mixing scratch mix sponge with or without emulsifier

mixing premix sponge

incorporating the following four sponge cake processing techniques:

spreading

rolling

layering

depositing

selecting, using and cleaning the baking equipment listed in the range of conditions, including controlling electrical hazards applicable to cleaning

finishing and decorating using the following four methods:

masking and combing

splitting and filling

applying piped finishes with cream or icing

garnishing

adding the following two creams and icings:

fresh cream or imitation cream

icing or basic butter cream

applying two of the following four piped finishing types:

rosettes

scrolls

rope

swirls

preparing and using the following three chocolate decorations:

writing simple celebration inscription

chocolate shavings

chocolate filigrees

documenting the production schedule, including the following six considerations:

timings

volume requirements

product processing requirements

recipe reformulation to minimise waste

finishing requirements for sponge cake products

bake parameters for sponge cake products.

Wastage that cannot be reused in production must be less than 5% of the weight of the original production schedule.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

workplace health and safety requirements, including personal protective equipment, applicable to producing sponge products

safe use and cleaning of baking equipment listed in the range of conditions, including electrical hazards

regulatory requirements for food safety applicable to producing sponge cake products

techniques and considerations for production scheduling, including:

timings

volume requirements

product processing requirements

recipe reformulation to minimise waste

finishing requirements for sponge cake products

bake parameters for sponge cake products

characteristics and storage requirements of ingredients used in sponge products production, including:

sponge premixes

eggs

sugar

cake fats and shortenings

cake flour

sponge emulsifiers

colouring and flavouring additives

fresh cream

imitation cream

compound chocolate

jams, conserves and spreads

determining the correct quantity of colourings and flavourings to match sponge batter size

selecting and preparing tins and trays to match sponge cake types, including:

greasing

spraying

papering

recipe formulations for sponge cake, including:

genoise

traditional

premix

emulsified

processes required for producing sponge cake batters, including:

mixing, including combination aerated and physically aerated

depositing

baking, including time and temperatures for sponge batter type and volume

de-panning without damage

cooling

packaging

preparation and applications of finishes, including:

aerating fresh creams

mixing icings

masking and combing sponges

piping decorations

melting compound chocolate

piping simple inscriptions for special occasion decorating

splitting and filling sponges

storage requirements for finished sponge cake products

techniques for testing bake of sponge by feel and sight

causes and corrective action for predictable and sometimes unpredictable sponge cake production problems, including:

machinery failure

environmental factors, including heat, humidity and cold

poor quality ingredients

techniques for disposing of waste

sponge products production terminology.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a commercial bakery or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment

equipment specified in the range of conditions

ingredients that meet the sponge cake product types required in the performance evidence

specifications:

recipes that meet the sponge cake products product types required in the performance evidence

end-product quality specifications that meet the sponge cake product types required in the performance evidence

food safety code regulatory requirements applicable to producing sponge cake products

timeframes:

according to the production schedule.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4