# Assessment Requirements for FBPRBK3011 Produce frozen dough products

Release: 1

Assessment Requirements for FBPRBK3011 Produce frozen dough products

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package version 1.0 |

# Performance Evidence

An individual demonstrating competency in this unit must satisfy all of the elements and performance criteria of this unit.

There must be evidence that the individual has produced frozen dough products according to a production schedule, including:

producing the following five frozen dough baked products:

laminated frozen dough product

basic bread frozen dough product

sweet yeast bread frozen dough product

specialty flour bread frozen dough product

chemical leavened frozen dough

incorporating the following four frozen dough processes:

ready-to-prove frozen dough

ready-to-bake frozen dough

par baked frozen dough

finishing frozen dough

using one of the following two simple fillings:

simple sweet filling

simple savoury filling

producing the following four product shapes:

baguette

round or long roll

croissant or danish

cut or portioned

producing the following three product styles:

decorated frozen dough

undecorated frozen dough

undecorated par baked dough

finishing the products using the following four techniques:

dusting

glazing

piping

scoring

selecting tins and trays appropriate for dough piece scale weight, volume and size

selecting, using and cleaning the baking equipment listed in the range of conditions, including controlling electrical hazards applicable to cleaning

documenting the production schedule, including the following seven considerations:

timings for freezing frozen and par baked dough

timings for thawing frozen and par baked dough

product processing requirements for frozen and par baked dough

volume requirements

recipe reformulation to minimise waste

freezing and thawing times

bake parameters for frozen product.

Wastage that cannot be reused in production must be less than 5% of the weight of the original production schedule.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

workplace health and safety requirements, including personal protective equipment, applicable to producing frozen dough products

safe use and cleaning of baking equipment listed in the range of conditions, including electrical hazards

regulatory requirements for food safety applicable to producing frozen dough products, including temperature control

techniques and considerations for production scheduling, including:

timings for freezing frozen and par baked dough

timings for thawing frozen and par baked dough

product processing requirements for frozen and par baked dough

volume requirements

recipe reformulation to minimise waste

freezing and thawing times

bake parameters for frozen product

characteristics and storage requirements of frozen dough types, including:

ready-to-prove frozen dough

ready-to-bake frozen dough

par baked frozen dough

chemically leavened frozen dough

functions and characteristics for frozen and par baked dough ingredients, including:

yeast levels

sugar inclusion

water and temperature of water

effects of salt

principles and techniques of freezing doughs, including:

snap freezing

storage stages, including freeze down and recovery

maintaining freezer temperatures

freezing times for dough types and sizes

optimal storage times

internal bake temperatures of par baked breads

storage patterns for frozen dough, including making efficient use of storage space, stock rotation and minimising temperature fluctuations

principles of thawing frozen and par baked doughs, including:

thawing times for dough types and sizes

consequences of slow or fast thawing

thawing rate for yeast doughs

types, functions, safe use, cleaning and freezing capacity of dough freezing equipment, including:

conventional freezers

blast freezers

handling requirements for made-up and proved doughs to preserve dough structure

impacts of factors on production of different frozen dough products, including:

finished dough temperature (FDT)

dough size

dough shape

yeast activity

processing frozen dough in cool phase

processing frozen dough in warm phase

techniques for calculating yields, adjusting recipes and measuring ingredients

causes and corrective action for predictable and sometimes unpredictable frozen dough product production problems, including:

frozen dough inconsistent temperature

frozen dough inconsistent moisture control

machinery failure

environmental factors, including heat, humidity and cold

poor quality ingredients

techniques for disposing of waste from frozen dough product production

frozen dough product production terminology.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

a commercial bakery or an environment that accurately represents workplace conditions

resources, equipment and materials:

personal protective equipment

equipment specified in the range of conditions

ingredients that meet the frozen dough product types required in the performance evidence

specifications:

recipes that meet the frozen dough product types required in the performance evidence

end-product quality specifications that meet the product types required in the performance evidence

food safety code regulatory requirements applicable to producing frozen dough products

timeframes:

according to the production schedule.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volumes, including Implementation Guides, are available at VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4