# Assessment Requirements for FBPSUG2015 Operate a high grade fugal station

Release: 1

Assessment Requirements for FBPSUG2015 Operate a high grade fugal station

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated a high grade fugal station in line with workplace contexts and demonstrated each of the following points at least once:

confirmed availability of equipment, services and massecuite to meet production requirements

selected, fitted and used personal protective equipment (PPE)

conducted pre-start checks and followed start-up procedure

started up and operated in both automatic and manual modes

safely operated, monitored and adjusted the high grade fugal distributor to the sugar bin and the A and B molasses storage tanks within required parameters

controlled dryer airflows to achieve target sugar temperature and moisture

controlled station throughput and A/B balance to meet pan stage throughput

performed an operational shutdown according to schedule or as indicated by equipment monitoring

conducted shift handover according to workplace procedure

taken corrective action in response to typical operating faults and product and process non-conformance.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

purpose and principles of high grade fugal station operation, including sugar drying and operation of batch and continuous fugals

factors that affect throughput and recovery, including the relationship between sugar pol and throughput and the balance between A and B fugals

the effect of massecuite quality on fugal operation and the effect of variation in operating parameters on the fugal output

equipment purpose and basic operating principles of high grade fugal equipment

significance and method of monitoring control points within the process

key health and safety requirements when operating a high grade fugal process:

hazards and associated control measures

correct use of PPE

procedures for responding to emergency situations

lock-out and tag-out procedures

shutdown sequence, including massecuite feed pumps and re-heaters

requirements of operational and long-term shutdown conditions

procedures and responsibility for reporting problems relating to the high grade fugal process

environmental and waste handling requirements and procedures relevant to the high grade fugal process

recording requirements and procedures relevant to high grade fugal process operations.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a sugar milling workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

PPE

high grade fugal station equipment

materials, including massecuite

routine maintenance tools

workplace information recording system

specifications:

workplace and operating procedures and related advice on equipment operation, including advice on safe work practices and environmental requirements

product and process specifications and operating parameters

cleaning procedures, sampling schedule and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4