# Assessment Requirements for FBPSUG2019 Operate a bagasse fuel supply system

Release: 1

Assessment Requirements for FBPSUG2019 Operate a bagasse fuel supply system

# Modification History

| Release | Comments |
| --- | --- |
| Release 1 | This version released with FBP Food, Beverage and Pharmaceutical Training Package Version 2.0. |

# Performance Evidence

An individual demonstrating competency must satisfy all of the elements and performance criteria in this unit.

There must be evidence that the individual has operated a bagasse fuel supply system in line with workplace contexts and demonstrated each of the following points at least once:

selected, fitted and used required personal protective equipment (PPE)

conducted pre-start checks of equipment, fuel and services

followed start-up procedures in both manual and automatic modes

safely operated, monitored and adjusted a bagasse fuel system and related auxiliary fuel system to supply a boiler

cleared fuel blockages or chokes throughout the system according to workplace procedures

applied fuel storage and/or reclamation procedures

applied procedures for test firing burners

removed, inspected, cleaned and replaced oil burners

carried out shift handover procedures

taken a fuel supply system off-line according to workplace procedures

taken corrective action in response to abnormal or unacceptable performance

reported fuel system operations and maintenance requirements

controlled work health and safety risks

safely shut down bagasse fuel supply system.

# Knowledge Evidence

An individual must be able to demonstrate the knowledge required to perform the tasks outlined in the elements and performance criteria of this unit. This includes knowledge of:

properties of bagasse and bagacillo, and impact of variation on combustion

impact of bagasse belt levels on system capacity to supply

principles of flame management systems, including understanding of when and how to purge boiler before relighting in a flame-out situation

purpose and operation of auxiliary fuel supply

functions and basic operating principles of fuel supply system, components and auxiliary equipment

fuel storage requirements relevant to a bagasse fuel system

fuel reclamation options and procedures

the effect of fuel quality and supply on boiler operation

bagasse fuel supply system layout to supply a boiler

relationship between viscosity and temperature for burner operation

purpose of atomising steam and/or air

purpose of purge cycle for the burner

relationship between fuel supply system and other processes

key health and safety requirements relevant to operating a bagasse fuel system:

hazards related to operation of fuel supply system, and associated control measures

correct use of PPE

methods used to render equipment safe to inspect, maintain and clean, including lock-out, tag-out and isolation procedures

procedures for responding to emergency situations, including emergency shutdown

handover and long-term shutdown and storage procedures

procedures and responsibility for reporting problems relevant to operating a bagasse fuel system

environmental and waste handling requirements and procedures relevant to operating a bagasse fuel system

recording requirements and procedures for operating a bagasse fuel system.

# Assessment Conditions

Assessment of skills must take place under the following conditions:

physical conditions:

skills must be demonstrated in a sugar milling workplace or an environment that accurately represents workplace conditions

resources, equipment and materials:

PPE

bagasse fuel supply equipment

fuel and services as required

maintenance tools

workplace information recording system

specifications:

operating procedures and related advice on equipment operation, including advice on safe work practices and environmental requirements

product and process specifications and operating parameters

workplace cleaning standards and procedures

workplace information recording procedures

sampling schedule and procedures.

Assessors of this unit must satisfy the requirements for assessors in applicable vocational education and training legislation, frameworks and/or standards.

# Links

Companion Volume Implementation Guides are found in VETNet: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=78b15323-cd38-483e-aad7-1159b570a5c4