# FDFCON2009A Operate a starch moulding process

Revision Number: 1

FDFCON2009A Operate a starch moulding process

# Modification History

Not applicable.

# Unit Descriptor

| Unit descriptor | This unit of competency covers the skills and knowledge required to set up, operate, adjust and shut down sugar moulded products using a starch moulding process. |
| --- | --- |

# Application of the Unit

| Application of the unit | This unit has application in a confectionery production environment. It typically targets the production worker responsible for applying basic operating principles to the operation and monitoring of starch moulding equipment and processes. When batch or product changeover procedures are part of this work process, the procedures should be used to customise the application of this unit. Where more detailed changeovers are carried out, FDFOP2011A Conduct routine maintenance, should be considered. |
| --- | --- |

# Licensing/Regulatory Information

Not applicable.

# Pre-Requisites

| Prerequisite units |  |  |
| --- | --- | --- |

# Employability Skills Information

| Employability skills | This unit contains employability skills. |
| --- | --- |

# Elements and Performance Criteria Pre-Content

| Elements describe the essential outcomes of a unit of competency. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the required skills and knowledge section and the range statement. Assessment of performance is to be consistent with the evidence guide. |
| --- | --- |

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| 1. Prepare the starch moulding equipment and process for operation | 1.1. Materials are confirmed and available to meet operating requirements 1.2. Starch is correctly conditioned for moulding 1.3. Cleaning and maintenance requirements and status are identified and confirmed 1.4. Starch mould is prepared to meet production requirements 1.5. Processing/operating parameters are selected for the depositor, starch moulding and starch removal equipment to meet safety and production requirements 1.6. Ingredients/mix is transferred to depositing equipment 1.7. Equipment performance is checked and adjusted as required 1.8. Pre-start checks are carried out as required by workplace requirements |
| 2. Operate and monitor the starch moulding and removal process | 2.1. The process is started and operated according to workplace procedures 2.2. Equipment is monitored to identify variation in operating conditions 2.3. Variation in equipment operation is identified and maintenance requirements are reported according to workplace reporting requirements 2.4. The process is monitored to confirm that specifications are met 2.5. Out-of-specification product/process outcomes are identified, rectified and/or reported to maintain the process within specification 2.6. The work area is maintained according to housekeeping standards 2.7. Work is conducted in accordance with workplace environmental guidelines 2.8. Workplace records are maintained according to workplace recording requirements |
| 3. Shut down the starch moulding process | 3.1. The appropriate shutdown procedure is identified 3.2. The process is shut down according to workplace procedures 3.3. Maintenance requirements are identified and reported according to workplace reporting requirements |

# Required Skills and Knowledge

REQUIRED SKILLS AND KNOWLEDGE

This section describes the skills and knowledge required for this unit.

Required skills

Ability to: access workplace information to identify starch moulding processing requirements select, fit and use personal protective clothing and/or equipment confirm supply of necessary materials and services, including confirming starch condition and mould type prepare product for depositing, such as ingredient addition, mixing and cooking to specification conduct pre-start checks, such as inspecting equipment condition to identify any signs of wear, selecting appropriate settings and/or related parameters, cancelling isolation or lock outs as required, confirming that equipment is clean and correctly configured for processing requirements, positioning sensors and controls correctly, ensuring any scheduled maintenance has been carried out, and confirming that all safety guards are in place and operational deposit and stamp starch moulds set up destarching and finishing equipment, such as blowing, oiling or sugaring clean starch and drying starch for re-use start, operate, monitor and adjust starch moulding process equipment to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, such as: the supply of liquid mass to hopper controlling weight of liquid deposited in mould monitoring pump stroke settings conducting visual inspections to identify faults, such as tailings and placement of liquid in mould monitor supply and flow of materials to and from the starch moulding process and equipment operation to confirm process remains within specification demonstrate depositor head changeovers take corrective action in response to out-of-specification results respond to and/or report equipment failure within level of responsibility locate emergency stop functions on equipment follow isolation and lock out/tag out procedures as required to take process and related equipment off-line in preparation for cleaning and/or maintenance within level of responsibility complete workplace records as required maintain work area to meet housekeeping standards use process control systems according to enterprise procedures collect samples and conduct tests according to enterprise procedures conduct routine maintenance according to enterprise procedures clean and sanitise equipment according to enterprise procedures use oral communication skills/language competence to fulfil the job role as specified by the organisation, including questioning, active listening, asking for clarification and seeking advice from supervisor work cooperatively within a culturally diverse workforce

Required knowledge

Knowledge of: purpose and basic principles of the starch moulding process, including an understanding of each stage: starch mould preparation product preparation and conditioning product depositing starch use and recovery cycle basic operating principles of equipment, such as main equipment components, status and purpose of guards, equipment operating capacities and applications, and the purpose and location of sensors and related feedback instrumentation services required and action to take if services are not available the flow of the starch moulding and depositing process and the effect of outputs on downstream processes quality requirements of starch moulds and materials for depositing and effect of variation on starch moulding process performance and outputs, including the moisture absorption characteristics of starch and impact on the process quality characteristics required of process outputs operating requirements and parameters and corrective action required where operation is outside specified operating parameters typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems methods used to monitor the starch moulding process, such as inspecting, measuring and testing as required by the process inspection or test points (control points) in the process and the related procedures and recording requirements contamination/food safety risks associated with the starch moulding process and related control measures common causes of variation and corrective action required occupational health and safety (OHS) hazards and controls requirements of different shutdowns as appropriate to the starch moulding process and workplace production requirements, including emergency and routine shutdowns and procedures to follow in the event of a power outage isolation, lock out and tag out procedures and responsibilities product/process changeover procedures and responsibilities procedures and responsibility for reporting production and performance information environmental issues and controls relevant to the starch moulding process, including waste/rework collection and handling procedures related to the process basic operating principles of process control, where relevant, including the relationship between control panels and systems and the physical equipment sampling and testing associated with process monitoring and control where relevant routine maintenance procedures where relevant cleaning and sanitation procedures where relevant

# Evidence Guide

| EVIDENCE GUIDE |  |
| --- | --- |
| The Evidence Guide provides advice on assessment and must be read in conjunction with the performance criteria, required skills and knowledge, range statement and the Assessment Guidelines for the Training Package. |  |
| Overview of assessment | Assessment must be carried out in a manner that recognises the cultural and literacy requirements of the assessee and is appropriate to the work performed. Competence in this unit must be achieved in accordance with food safety standards and regulations. |
| Critical aspects for assessment and evidence required to demonstrate competency in this unit | Evidence of ability to: conduct pre-start checks on machinery used for starch moulding start, operate, monitor and adjust process equipment to achieve required quality outcomes take corrective action in response to typical faults and inconsistencies complete workplace records as required apply safe work practices and identify OHS hazards and controls safely shut down equipment apply food safety procedures to work practices. |
| Context of and specific resources for assessment | Assessment must occur in a real or simulated workplace where the assessee has access to: personal protective clothing and equipment work procedures, including advice on safe work practices, food safety, quality and environmental requirements information on equipment capacity and operating parameters recipe/batch instructions specifications, control points and processing parameters process and related equipment and services, including moulds, starch stamping and mould preparation and depositing equipment materials to be processed, including starch for moulding and ingredients/mix to be deposited sampling schedules and test procedures and equipment as required documentation and recording requirements and procedures cleaning procedures, materials and equipment as required. |
| Method of assessment | This unit should be assessed together with core units and other units of competency relevant to the function or work role: Examples could be: FDFOP2004A Clean and sanitise equipment FDFOP2011A Conduct routine maintenance FDFOP2013A Apply sampling procedures FDFOP2030A Operate a process control interface MSL973001A Perform basic tests. |
| Guidance information for assessment | To ensure consistency in one's performance, competency should be demonstrated on more than one occasion over a period of time in order to cover a variety of circumstances, cases and responsibilities, and where possible, over a number of assessment activities. |

# Range Statement

| RANGE STATEMENT |  |
| --- | --- |
| The range statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. Bold italicised wording, if used in the performance criteria, is detailed below. Essential operating conditions that may be present with training and assessment (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) may also be included. |  |
| Policies and procedures | Work is carried out according to company policies and procedures, regulatory and licensing requirements, legislative requirements, and industrial awards and agreements |
| Legislative requirements | Legislative requirements are typically reflected in procedures and specifications. Legislation relevant to this industry includes: the Food Standards Code, including labelling, weights and measures legislation legislation covering food safety, environmental management, OHS, anti-discrimination and equal opportunity |
| Workplace information | Workplace information may include: standard operating procedures (SOPs) specifications production schedules and instructions manufacturers' advice standard forms and reports |
| Starch moulded products | Typical starch moulded products include: gums and jellies marshmallows caramels |
| Materials | Materials may include: starch low or high boil syrups |
| Equipment | Typical equipment includes: boiling pans (batch) moguls (continuous process) drying room and starch cleaning equipment Related processes may include: mixing and cooking of ingredients prior to depositing |
| Operation of equipment and processes | Operation of equipment and processes may require: the use of process control panels and systems |
| Shutdown procedures | Shutdown procedures may include: cleaning (in some cases cleaning may be carried out by a dedicated cleaning crew) |
| Services | Services may need to be confirmed. These depend on the nature of the process. Typical examples include: power steam water vacuum compressed and instrumentation air |

# Unit Sector(s)

| Unit sector | Confectionery |
| --- | --- |

# Competency field

| Competency field |  |
| --- | --- |

# Co-requisite units

| Co-requisite units |  |  |
| --- | --- | --- |