# FDFPH2003A Operate an extraction process

Revision Number: 1

FDFPH2003A Operate an extraction process

# Modification History

Not applicable.

# Unit Descriptor

| Unit descriptor | This unit of competency covers the skills and knowledge required to set up, operate, adjust and shut down processes used to extract phytochemicals from plant material using solvents. |
| --- | --- |

# Application of the Unit

| Application of the unit | This unit applies to production operators working in the pharmaceutical sector. This person would typically work within defined Good Manufacturing Practice (GMP) programs and procedures. |
| --- | --- |

# Licensing/Regulatory Information

Not applicable.

# Pre-Requisites

| Prerequisite units |  |  |
| --- | --- | --- |

# Employability Skills Information

| Employability skills | This unit contains employability skills. |
| --- | --- |

# Elements and Performance Criteria Pre-Content

| Elements describe the essential outcomes of a unit of competency. | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the required skills and knowledge section and the range statement. Assessment of performance is to be consistent with the evidence guide. |
| --- | --- |

# Elements and Performance Criteria

| ELEMENT | PERFORMANCE CRITERIA |
| --- | --- |
| 1. Prepare the extraction equipment and process for operation | 1.1. Materials are confirmed, blended and prepared to meet production requirements 1.2. Workplace documentation relevant to work area activities is identified and followed 1.3. The required facilities, storage, equipment and personnel are available 1.4. Line clearance procedures have been carried out 1.5. Procedures to eliminate or control the risk of cross-contamination are followed 1.6. Material is loaded into percolator and solvents are added to specification |
| 2. Operate and monitor the extraction process | 2.1. The extraction process is monitored to confirm that specifications are met 2.2. Out-of-specification product/process is identified, rectified and/or reported to maintain the process within specification 2.3. The work area is maintained according to housekeeping standards 2.4. Work is conducted according to environmental standards 2.5. Workplace documentation is maintained according to workplace reporting requirements |
| 3. Shut down the extraction process | 3.1. The process is shut down according to workplace procedures and work practices 3.2. Maintenance requirements are identified and reported according to workplace reporting requirements |

# Required Skills and Knowledge

REQUIRED SKILLS AND KNOWLEDGE

This section describes the skills and knowledge required for this unit.

Required skills

Ability to: select, fit and use personal protective clothing and/or equipment conduct pre-start checks, such as inspecting equipment condition to identify any signs of wear, confirming that equipment is clean and correctly configured for processing requirements, positioning sensors and controls correctly, ensuring any scheduled maintenance has been carried out, and placing sand filters/scourers in base of percolators where required start, operate, monitor and adjust process to achieve required outcomes, including monitoring control points and conducting inspections as required to confirm process remains within specification, such as: density of herbs in percolator, including re-packing of percolator as required rate/amount of solvent addition extract collection and yield take corrective action in response to out-of-specification results respond to and/or report equipment failure within level of responsibility demonstrate batch/product changeovers including line clearance procedures sort, collect, treat, recycle or dispose of waste clean and sanitise equipment as required as required complete workplace records as required maintain work area to meet housekeeping standards use oral communication skills/language competence to fulfil the job role as specified by the organisation, including questioning, active listening, asking for clarification and seeking advice from supervisor work cooperatively within a culturally diverse workforce

Required knowledge

Knowledge of: purpose and principles of each stage of the extraction process, including the effect of herb density on filtration and packing process required for different types of herbs basic operating principles of equipment, including main equipment components and equipment operating capacities and applications quality requirements of materials and the effect of variation on the extraction process process specifications, procedures and operating parameters for different products/materials operating requirements and parameters and corrective action required where operation is outside specified operating parameters typical equipment faults and related causes, including signs and symptoms of faulty equipment and early warning signs of potential problems methods used to monitor the extraction process, such as inspecting, measuring and testing as required by the process, and the ability to calculate yields contamination/food safety risks associated with the extraction process common causes of variation and corrective action required occupational health and safety (OHS) hazards and controls, including the risks involved with the use of solvents, such as ethanol, and the limitations of protective clothing and equipment used extraction process shutdown and changeover procedures and responsibilities environmental issues and controls relevant to the extraction process, including waste collection and handling procedures related to the process cleaning and sanitation procedures workplace documentation and authorisation procedures

# Evidence Guide

| EVIDENCE GUIDE |  |
| --- | --- |
| The Evidence Guide provides advice on assessment and must be read in conjunction with the performance criteria, required skills and knowledge, range statement and the Assessment Guidelines for the Training Package. |  |
| Overview of assessment | Assessment may occur in a real or simulated workplace context. Where a simulated environment is used, it should provide access to workplace documentation, and a typical range of extraction solvents, material and equipment. |
| Critical aspects for assessment and evidence required to demonstrate competency in this unit | Evidence of ability to: prepare the extraction process for operation, including following line clearance procedures load materials and solvents to maximise extract collection monitor the extraction process maintain all necessary records. |
| Context of and specific resources for assessment | Assessment must occur in a real or simulated workplace where the assessee has access to: workplace documentation relating to concentration process and procedures. |
| Method of assessment | Assessors must be satisfied that the person can consistently perform the unit as a whole, including all elements, performance criteria, and required skills and knowledge. A holistic approach should be taken to the assessment. Assessment of this unit would typically involve observation of the operator preparing, operating and shutting down the extraction process to meet production requirements. Observation should be sufficient to confirm consistent performance. |
| Guidance information for assessment | To ensure consistency in one's performance, competency should be demonstrated on more than one occasion over a period of time in order to cover a variety of circumstances, cases and responsibilities, and where possible, over a number of assessment activities. |

# Range Statement

| RANGE STATEMENT |  |
| --- | --- |
| The range statement relates to the unit of competency as a whole. It allows for different work environments and situations that may affect performance. Bold italicised wording, if used in the performance criteria, is detailed below. Essential operating conditions that may be present with training and assessment (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) may also be included. |  |
| Policies and procedures | Work activities are carried out according to company policies and procedures, regulatory and licensing requirements, legislative requirements and industrial awards and agreements |
| Legislative requirements | Legislative requirements are typically reflected in procedures and specifications. Legislation relevant to this industry includes: legislative and licensing requirements Therapeutic Goods Act weights and measures legislation legislation relating to OHS, environmental management, equal opportunity and affirmative action, industrial awards and agreements |
| Workplace documentation | Workplace documentation relevant to work area activities include: specifications manufacturing formulae processing instructions batch production records standard operating procedures (SOPs) OHS information, including material safety data sheets (MSDS) |
| Equipment | Equipment may include: blenders/mixers percolators tamping rods collection vessels |
| Extraction process | Solvents used in the extraction process may be: aqueous and/or alcohol based (temperature may or may not be applied during extraction process) |

# Unit Sector(s)

| Unit sector | Pharmaceutical manufacturing |
| --- | --- |

# Competency field

| Competency field |  |
| --- | --- |

# Co-requisite units

| Co-requisite units |  |  |
| --- | --- | --- |